Unit 2 ELEC2 - Further Electronics

- Capacitors

Candidates should be able to:

- recall that a capacitor is capable of storing electrical charge and electrical energy;

- recall that a capacitor will block a direct current but will allow the passage of an alternating current;

- recall that the unit of capacitance is the farad and that practical capacitors are usually measured in pF, nF and µF;

- calculate the combined capacitance of capacitors connected in series and parallel combinations;

- select appropriate capacitors given data on maximum working voltage, polarisation and leakage current.

dc RC networks

Candidates should be able to:

- explain the meaning of and calculate the value of the time constant for RC circuits;

- recall that after one time constant:
	for a charging capacitor, V = 0.63Vs;		for a discharging capacitor, V = 0.37Vs;
	where Vs is the supply voltage and V is the voltage across the capacitor;

- recall that:
	after 0.69RC, V = 0.5Vs;
	after 5RC for a charging capacitor, V  Vs;	after 5RC for a discharging capacitor, V  0;

- sketch graphs of voltage against time for a capacitor charging and discharging.

- Timing subsystems

Candidates should be able to:

- recall that a monostable circuit has one stable output state and one unstable output state;

- recognise, draw and use the circuit diagram of a monostable based on a 555 timer circuit;

- describe the operation of a monostable based on a 555 timer;

- calculate the time period of such a monostable using T = 1.1RC;

- recall that an astable circuit has no stable output states but continually changes;

- recognise, draw and use the circuit diagram of an astable based on a 555 timer circuit;

- describe the operation of an astable based on a 555 timer;

- calculate the time, tL, that the output is low using tL = 0.7RBC;

- calculate the time, tH, that the output is high using tH = 0.7(RA+RB)C;

- calculate the output frequency using F= 1.44C(RA + 2 RB).

- Sequential logic subsystems

Candidates should be able to:

- recall the circuit diagram of a bistable latch based on NAND gates and describe its operation and function;

- recall the symbol for a rising edge triggered D-type flip-flop and describe its operation and function;

- recall that in a shift register information is passed along from one element to the next on each clock pulse;

- recall how rising edge triggered D-type flip-flops can be used to form a shift register and describe its operation and applications.

- Counter subsystems

Candidates should be able to:

- describe the use of feedback to make a rising edge triggered D-type flip-flop divide by 2;

- design 4-bit up or down counters based on rising edge triggered D-type flip-flops, and draw timing diagrams for these counters;

- design 4-bit modulo-N counters based on rising edge triggered D-type flip-flops, and draw timing diagrams for these counters;

- convert a 4-bit binary number to decimal or hexadecimal notation;

- describe the use of a BCD or hexadecimal decoder with a seven segment display.

- The Operational Amplifier

Candidates should be able to:

- recall the properties of an ideal op-amp;

- recall that for a real op-amp, the product of voltage gain and bandwidth is a constant;

- recall that negative feedback is used to reduce the overall voltage gain of an op-amp amplifier subsystem.

- Amplifier subsystems

Candidates should be able to:

- use the formula  voltage gain=VoutVin;

- recall and use the definition of bandwidth of an amplifier as the frequency range over which the voltage gain is within 70% of the maximum;

- draw and recognise the inverting op-amp amplifier circuit and describe its applications;

- use the formula  VoutVin = − RfRin;

- recall that the input resistance is equal to the value of the input resistor, and that the circuit has a virtual earth point;

- draw and recognise a summing op-amp amplifier circuit and describe its applications;

- use the formula  Vout=−RfV1R1+V2R2+VR3;

- recall that the input resistance of each input is equal to the value of its input resistor, and that the circuit has a virtual earth point;

- draw and recognise the single op-amp difference amplifier circuit and describe its applications;

- use the formula  Vout=V+ − V−RfR1;

- recall that the input resistance of each input is different and comparatively low;

- draw and recognise the non-inverting op-amp amplifier circuit and describe its applications;

- use the formula  VoutVin = 1 + RfR1;

- recall that the input resistance is equal to that of the op-amp;

- draw and recognise the voltage follower op-amp amplifier circuit and describe its applications;

- recall that the voltage gain of a voltage follower is 1, but that the current and power gain can be very large;

- recall that the input resistance is equal to the resistance of the op-amp.

- Power amplifier subsystems

Candidates should be able to:

- use the formula  Power Gain=PoutPin;

- recall and use the definition of bandwidth of an amplifier as the frequency range over which the power gain is within 50% of the maximum;

- draw and recognise the enhancement mode MOSFET (both n- and p-channel) source follower amplifier circuits and describe their applications;

- estimate the power dissipated in a source follower and describe methods for removing the excess heat generated;

- draw and recognise the push-pull amplifier circuit using p- and n-channel enhancement mode MOSFETs and describe its operation and applications;

- describe the common types of distortion associated with push-pull amplifier subsystems (cross-over and saturation/clipping) and how they can be reduced;

- describe the advantages of push-pull amplifier subsystems over single ended output subsystems;

- estimate the maximum power output from a push-pull amplifier subsystem.