NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

COUNTERS

PACKAGE

(1).	Divide by 2 using a D-type flip-flop

(2)	Conversion of 4-bit binary numbers to decimal and Hexadecimal notation

(3).	4-bit down-counters

(4).	4-bit up-counters

(5)	Designing 4-bit modulo-N counters

(6)	Using BCD and Hexadecimal decoders with a 7-segment display

(1).	Divide by 2 using a D-type Flip-Flop.

The aim of this assignment is to understand how a D-type flip-flop can be used to divide by two.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a divide by 2 D-type flip-flop circuit,

Explain how a divide by 2 D-type flip-flop circuit works.

Know some applications of a divide by 2 D-type flip-flop circuit.

You will need to use the Support Booklet.  The D-type flip-flop will be contained in a 4013 IC.

1).	(a)	Draw a diagram to show the pin-out of the 4013 IC.

(b)	A D-type flip-flop wired to divide by two is shown in the circuit diagram below.

S and R are both connected to logic 0 and the CK input is connected to 0V with a

100k resistor.

(c)	(i)	If Q is logic 0, what will be the state of Q and D?  ...................................

(c)	(ii)	Explain what will happen to Q when the CK input momentarily becomes 				logic 1.

..................................................................................................................................

..................................................................................................................................

(c)	(iii)	Explain what will now happen to Q when the CK input again momentarily 			becomes logic 1.

..................................................................................................................................

..................................................................................................................................

(d)	Construct the divide by 2 D-type flip-flop circuit on protoboard.  Use the protoboard 		layout sheet below to help you.  Remember to connect all unused inputs to 0V.

Connect your circuit to a 5V power supply.

Connect the input to a signal generator producing 5V pulses at 1kHz.

Connect an oscilloscope so that you can display both the input and output waveforms.

Complete the diagram below.

(e)	Use the support booklet to explain how the circuit works.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(2).	Binary, Decimal and Hexadecimal numbers.

The aim of this assignment is to understand the Binary and Hexadecimal numbers and to be able to convert 4-bit binary numbers to decimal and hexadecimal notation.

You will need to use the Support Booklet.

1).	(a)	What is the number base for binary numbers?

..................................................................................................................................

(b)	Explain why binary number notation is so useful for digital electronics.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	What is the number base for hexadecimal numbers?

..................................................................................................................................

(d)	Explain why hexadecimal number notation is so useful for computer systems.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(e)	What is the number base for decimal numbers?

..................................................................................................................................

(f)	What is the maximum value of a 4-bit binary number?

..................................................................................................................................

With binary numbers, explain what is meant by the term:-

(g)	most significant bit

..................................................................................................................................

(h)	least significant bit

..................................................................................................................................

2).	Complete the following table which shows the conversion between binary, decimal 		and hexadecimal numbers.

| BINARY D C B A | DECIMAL | HEXADECIMAL |
| --- | --- | --- |
|  | 0 |  |
|  |  | 1 |
|  | 2 |  |
|  | 3 |  |
| 0 1 0 0 |  |  |
|  | 5 |  |
|  |  | 6 |
| 0 1 1 1 |  |  |
|  | 8 |  |
|  |  | 9 |
| 1 0 1 0 |  |  |
|  | 11 |  |
|  |  | C |
| 1 1 0 1 |  |  |
|  | 14 |  |
|  |  | F |

While it is possible to "work out" each of the numbers in the table above, since only 4-bit numbers are needed for this module it is probably easier to learn the table.

(3).	Four Bit Down Counters.

The aim of this assignment is to understand how D-type flip-flops can be cascaded together to form binary counters.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a four bit down counter,

Explain how a four bit down counter works,

Draw the timing diagram of a four bit down counter.

You will need to use the Support Booklet.  The four D-type flip-flops are contained in two 4013 ICs.

1).	(a)	On protoboard construct the circuit shown in the diagram below.

Use the protoboard diagram below to help you.

Remember to connect all unused inputs to 0V.

Connect your circuit to a 5V power supply.

Connect the input to a signal generator producing 5V pulses at 1kHz.

Connect an oscilloscope so that you can display both the input and the Q0 output waveforms.

Complete the Q0 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q0 and Q1 output waveforms.

Complete the Q1 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q1 and Q2 output waveforms.

Complete the Q2 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q2 and Q3 output waveforms.

Complete the Q3 waveform on the diagram below.

(b)	Complete the row labelled "decimal" with the number represented by the 4-bit binary 		number.

(c)	Explain why this circuit is known as a down counter.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	How can a 4-bit up counter be constructed?

..................................................................................................................................

..................................................................................................................................

(4).	Four Bit Up Counters.

The aim of this assignment is to understand how D-type flip-flops can be cascaded together to form binary counters.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a four bit up counter,

Explain how a four bit up counter works,

Draw the timing diagram of a four bit up counter.

You will need to use the Support Booklet.  The four D-type flip-flops are contained in two 4013 ICs.

1).	(a)	On protoboard construct the circuit shown in the diagram below.

Use the protoboard diagram below to help you.

Remember to connect all unused inputs to 0V.

Connect your circuit to a 5V power supply.

Connect the input to a signal generator producing 5V pulses at 1kHz.

Connect an oscilloscope so that you can display both the input and the Q0 output waveforms.

Complete the Q0 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q0 and Q1 output waveforms.

Complete the Q1 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q1 and Q2 output waveforms.

Complete the Q2 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q2 and Q3 output waveforms.

Complete the Q3 waveform on the diagram below.

(b)	Complete the row labelled "decimal" with the number represented by the 4-bit binary 		number.

(c)	Explain why this circuit is known as an up counter.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	How can a circuit that counts up to 9 and then resets itself, be constructed?

..................................................................................................................................

..................................................................................................................................

(5).	Four Bit Modulo-N Counters.

The aim of this assignment is to understand how D-type flip-flops can be cascaded together to form counters that count up to numbers other than multiples of 2.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a four bit modulo-N counter,

Explain how a four bit modulo-N counter,

Draw the timing diagram of a four bit modulo-N counter.

You will need to use the Support Booklet.  The four D-type flip-flops are contained in two 4013 ICs and the AND gate can be made from two NAND gates from a 4011 IC.

The most important of these modulo-N counters is one that counts up to 9 and then resets 	itself on the tenth input pulse.

Such a counter is known as a Binary Coded Decimal (BCD) counter.

1).	(a)	Construct the circuit for the circuit diagram shown below on protoboard.

Use the protoboard diagram below to help you.

Remember to connect all unused inputs to 0V.

Connect your circuit to a 5V power supply.

Connect the input to a signal generator producing 5V pulses at 1kHz.

Connect an oscilloscope so that you can display both the input and the Q0 output waveforms.

Complete the Q0 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q0 and Q1 output waveforms.

Complete the Q1 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q1 and Q2 output waveforms.

Complete the Q2 waveform on the diagram below.

Then move the oscilloscope probes so that you can display the Q2 and Q3 output waveforms.

Complete the Q3 waveform on the diagram below.

(b)	Complete the row labelled "decimal" with the number represented by the 4-bit binary 		number.

(c)	Examine the waveform from Q3.  What is its mark to space ratio?

..................................................................................................................................

..................................................................................................................................

(d)	How would it be possible to produce a divide by 10 waveform with a mark to space 		ratio of 1:1?

..................................................................................................................................

..................................................................................................................................

2).	(a)	Complete the circuit diagram below so that the output from Q3 is divided by 10 and 		has a mark to space ratio of 1:1.

(b)	Complete the timing diagram below for this circuit.

(c)	Explain why this circuit is able to produce an output with a mark to space ratio

of 1:1.

..................................................................................................................................

..................................................................................................................................

3).	(a)	Complete the circuit diagram below so that the output from Q3 is divided by 12 and 		has a mark to space ratio of 1:1.

(b)	Complete the timing diagram below for this circuit.

(c)	Explain why this circuit is able to produce an output with a mark to space ratio

of 1:1.

..................................................................................................................................

..................................................................................................................................

4).	(a)	Complete the circuit diagram below so that the circuit counts to 12 and then resets.

(b)	Complete the timing diagram below for this circuit.

(c)	Suggest a use for a "12 counter".

..................................................................................................................................

..................................................................................................................................

(6).	BCD and Hexadecimal decoders with a 7-segment display

The aim of this assignment is to understand how numbers can be displayed on a 7-segment display.

Normally you will be able to use an IC which will convert a 4-bit binary number directly to a form suitable for operating a 7-segment display eg 4511 and the 7447 ICs.  The section on Further Boolean algebra will explore building logic circuits to decode 4-bit numbers directly.

1).	Shade in the display in the diagrams below to show how the 4-bit binary numbers are 		translated to a 7-segment output.

Binary		...........................................

Decimal	.................0........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................1........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................2........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................3........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................4........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................5........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................6........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................7........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................8........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................9........................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................10......................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................11......................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................12......................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................13......................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................14......................

Hexadecimal	...........................................

Binary		...........................................

Decimal	.................15......................

Hexadecimal	...........................................

CIRCUIT AND TIMING DIAGRAM FOR……………………………………………………….

TIMING DIAGRAM