NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

POWER

AMPLIFIER

PACKAGE

(1).	Source follower

(2)	Push-pull MOSFET amplifiers

(3).	Cross-over distortion and clipping

(4).	Power output and efficiency

(5)	Heatsinks

(1).		Source follower.

The aim of this assignment is to revise MOSFETs and then to investigate and understand the source follower as a power amplifier.  You will need to use the Support Booklet.

1).	(a)	Complete the labelling of the diagram below.

(b)	List four important properties of MOSFETs

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Explain what is meant by rds and state a typical value.

..................................................................................................................................

..................................................................................................................................

(d)	Explain what is meant by gm and state a typical value.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

2).	(a)	Investigate how the drain current varies with the gate to source voltage for
		an IRF630 MOSFET.

Construct the circuit on protoboard and use a 12V power supply.

Complete the table with your measurements.

| Gate to Source Voltage (V) | Drain Current (mA) |
| --- | --- |
|  | 0 |
|  | 20 |
|  | 40 |
|  | 60 |
|  | 80 |
|  | 100 |
|  | 120 |
|  | 140 |
|  | 160 |
|  | 180 |
|  | 200 |

(b)	Plot a graph of your results on the grid below.

3).	(a)	Assemble the source follower circuit, shown in the diagram below, on your 			protoboard.

Investigate how the output voltage (Vout) varies with the input voltage (Vin).

Record your results in the table below.

| Input Voltage (Vin) | Output Voltage (Vout) |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |

(b)	Plot a graph of your results on the grid below.

(c)	Describe the relationship between Vout and Vin.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	What is the voltage gain of a source follower?

..................................................................................................................................

..................................................................................................................................

(e)	Explain your answer in terms of negative feedback.

..................................................................................................................................

..................................................................................................................................

(f)	What is a source follower able to provide for a circuit?

..................................................................................................................................

..................................................................................................................................

(g)	What is the input resistance of a source follower?

..................................................................................................................................

..................................................................................................................................

4).	(a)	Assemble the source follower circuit, shown in the diagram below, on your 			protoboard.

Connect Vin to a signal generator with a 1kHz, 5V amplitude sine wave output.

Display the input and output voltages waveforms on an oscilloscope.

Record your displays on the diagram below.  Label the waveforms.

(b)	State how the output waveform differs from the input waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Explain why the amplitude of the output waveform is less than the input waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	Suggest a way of overcoming this difficulty and also a way of producing the negative 		half cycles of the waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(2).		Push-pull MOSFET amplifiers.

The aim of this assignment is to understand how a n-channel and a p-channel MOSFET can be used together to provide power amplification.  You will need to use the Support Booklet.

1).	(a)	Assemble the source follower circuit, shown in the diagram below, on your 			protoboard.

Connect Vin to a signal generator with a 1kHz, 5V amplitude sine wave output.

Display the input and output voltages waveforms on an oscilloscope.

Record your displays on the diagram below.  Label the waveforms.

(b)	State how the output waveform differs from the input waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Explain why the amplitude of the output waveform is less than the input waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	Describe the output waveform as the input waveform changes from positive to 			negative.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(e)	Explain what causes this distortion to the waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(f)	Suggest ways to minimise this distortion.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(3).		Cross-over distortion and clipping.

The aim of this assignment is to understand the cause and ways to minimise the two most severe types of distortion associated with push-pull amplifiers.  You will need to use the Support Booklet.

Cross-over distortion occurs as the waveform passes from positive to negative and vice-	versa.  It is caused by the non-linearity of the MOSFETs for small values of Vgs.  This 	distortion can be minimised by applying a "bias voltage" to each of the MOSFETs so that 	they are no longer operating in the non-linear region of the MOSFETs' characteristic curves.

1).	(a)	Assemble the source follower circuit, shown in the diagram below, on your 			protoboard.

Connect Vin to a signal generator with a 1kHz, 5V amplitude sine wave output.

Display the input and output voltages waveforms on an oscilloscope.

Record your displays on the diagram below.  Label the waveforms.

(b)	Describe the output waveform, commenting particularly on region where cross-over 		distortion occurs.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Reduce the power supply voltage to +5V and increase the amplitude of the input 			waveform to 8V.  Draw the waveforms observed onto the diagram below.

(d)	Describe the output waveform.

..................................................................................................................................

..................................................................................................................................

(e)	Explain the reason for this distortion and state its common names.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

2).	(a)	The voltage gain of the push-pull amplifiers considered so far has been only 1.  To 		provide additional voltage gain, an op-amp can be used.  If the output MOSFETs are 		included into the feedback network, then distortion will be reduced.  Push pull 			MOSFET amplifiers can deliver considerable power to a low resistance load eg a 			loudspeaker.

Assemble the source follower circuit, shown in the diagram below, on your 			protoboard.

Connect Vin to a signal generator with a sine wave output.

Investigate the voltage gain, frequency response and distortion of the circuit.

Display the input and output voltages waveforms on an oscilloscope.

Record your displays on the diagram below.  Label the waveforms.

(b)	What is the overall voltage gain of the amplifier?  Explain your answer.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(4).		Power output and Efficiency.

The aim of this assignment is to understand how to measure and calculate the power output from an amplifier and the efficiency of the system.  You will need to use the Support Booklet.

1).	(a)	What measurements would you make in order to determine the maximum output 			power of an amplifier?

..................................................................................................................................

..................................................................................................................................

(b)	How would you use your measurements to calculate the maximum output power?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	An amplifier delivers a maximim peak voltage of 8V across a 4 speaker.

Calculate the maximum output power.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	Many modern amplifiers (especially computer speaker systems) are rated as

XX watts PMPO.  What do the initials PMPO stand for?

..................................................................................................................................

..................................................................................................................................

(e)	Explain why the PMPO power output has little relevance to the true power output of 		an amplifier system.

..................................................................................................................................

..................................................................................................................................

(f)	An amplifier operates from +12V supply and has a 4 speaker.

Calculate the maximum theoretical output power.

..................................................................................................................................

..................................................................................................................................

2).	(a)	Explain what measurements you would make to determine the maximum power 			input to an amplifier.

..................................................................................................................................

..................................................................................................................................

(b)	Explain how you would use your measurements to calculate the maximum power 			input.

..................................................................................................................................

..................................................................................................................................

(c)	An amplifier has a supply voltage of +35V and a maximum input current

of 2.1A rms.  What is the maximum input power?

..................................................................................................................................

..................................................................................................................................

3).	(a)	Explain what is meant by the efficiency of an amplifier.

..................................................................................................................................

..................................................................................................................................

An amplifier has a maximum output voltage of 30V across an 8 speaker.  Under 		these conditions the amplifier supply voltage is +35V and a the input current is

2.1A rms.  Calculate:-

(b)	the power output

..................................................................................................................................

..................................................................................................................................

(c)	the power input

..................................................................................................................................

..................................................................................................................................

(d)	the efficiency.

..................................................................................................................................

..................................................................................................................................

(e)	What happens to the energy that does not get to the speaker?

..................................................................................................................................

..................................................................................................................................

(f)	Suggest reasons why an amplifier is not 100% efficient.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

4).	(a)	Explain what is meant by a "Bridged Amplifier".

..................................................................................................................................

..................................................................................................................................

(b)	Draw a system diagram of a Bridged amplifier.

(c)	Explain the advantages to be gained by using a Bridged amplifier.

..................................................................................................................................

..................................................................................................................................

(d)	A Bridged amplifier is used in a car.  What is the theoretical maximum output power 		that it can deliver to a 4 loudspeaker?

..................................................................................................................................

..................................................................................................................................

(5).		Heatsinks.

The aim of this assignment is to understand the need for heatsinks and their design features.  You will need to use the Support Booklet.

1).	(a)	Explain the function of a heatsink.

..................................................................................................................................

..................................................................................................................................

(b)	Why are heatsinks essential in many electronic systems.

..................................................................................................................................

..................................................................................................................................

(c)	State three ways that heat can be transferred and explain how the design of a heatsink		maximises this transfer.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	How is a heatsink rated?

..................................................................................................................................

..................................................................................................................................

(e)	A power of 7W is dissipated in a MOSFET.  The maximum temperature that the 			MOSFET can withstand is 100oC.  If the maximum ambient temperature is 35oC, 			calculate the rating for a suitable heatsink.

..................................................................................................................................

..................................................................................................................................

Predicted results.

| Gate to Source Voltage (V) | Drain Current (mA) |
| --- | --- |
| 0 | 0 |
| 3.36 | 20 |
| 3.37 | 40 |
| 3.39 | 60 |
| 3.42 | 80 |
| 3.45 | 100 |
| 3.48 | 120 |
| 3.51 | 140 |
| 3.54 | 160 |
| 3.57 | 180 |
| 3.60 | 200 |

| Input Voltage (Vin) | Output Voltage (Vout) |
| --- | --- |
| 0 | 0 |
| 1 | 0 |
| 2 | 0 |
| 3 | 0.2 |
| 4 | 1.0 |
| 5 | 1.8 |
| 6 | 2. 8 |
| 7 | 3.7 |
| 8 | 4.7 |
| 9 | 5.7 |
| 10 | 6.6 |