ELEC4		Programmable Control Systems.

These notes should be read in conjunction with the specification.

Control Systems.

This section considers Control systems in general and candidates are expected to be able to analyse a given control system into the main functional blocks as shown in the diagram in the specification.

Control systems can be categorised as open loop, when no information is available from the output indicating whether an action has been completed correctly, or closed loop systems, when information from the output is available.  This information can be combined with the input to the control system in two ways:-

1).	it can either be subtracted from the input, negative feedback, resulting in the output being directly controlled by the input,

2).	it can reinforce the input, positive feedback, leading to the output usually only having two possible states.

Candidates should analyse control systems, electronic and non-electronic that are familiar to them, identifying both open and closed loop systems together with whether negative or positive feedback is involved.

Microprocessor subsystems.

Almost all industrial and commercial electronic systems contain microprocessor control systems.  The reason is obvious - it reduces costs:-

the functionality of a piece of equipment can be readily modified by a manufacturer simply by upgrading the software within the microprocessor controller,

the same microprocessor controller can be used in a very wide variety of equipment.

It is therefore vital that all students of electronics are exposed to microprocessor control systems and their use.

While there are many economic and social benefits of microprocessor systems, candidates should also be aware of possible adverse effects of the dependency on such systems.

Although most microprocessor control systems are now fabricated onto a single IC, specialist systems will use discrete elements, and candidates should be aware of the basic components of a microprocessor control system and the function of each element.

Microprocessor systems with RAM, ROM, I/O ports, buses etc all fabricated into an IC are known as microcontrollers.  As technology advances, the number of microcontrollers available increases year on year as does the functionality offered by each device.  It would not be possible to keep up to date with such developments within an examination specification and so it was decided to focus on the fundamental concepts of microcontrollers and develop skills that are transferable to future devices.  As a result, this specification assumes a generic microcontroller with a Harvard architecture and the following specification:-

- a clock speed of between 1 and 20MHz;

- an accumulator or working register, W, through which all calculations are performed;

- a program counter, PC;

- three 8-bit bi-directional ports - PORTA, PORTB and PORTC;

- three data direction registers TRISA, TRISB and TRISC, to determine whether the bits of each port are inputs or outputs;

- for the data direction registers, if a bit is set to 1 then the port bit is an input, if the bit is set to 0 then the port bit is an output;

- a status register for which bit 0 is the carry flag, C, and bit 2 is the zero flag, Z;

- a clock prescaler, which can be set to divide the clock frequency by 2 to 256;

- an 8-bit timer register, TMR, which is incremented on each rising edge of the clock prescaler pulse and which sets bit 1 of the status flag when it is 0.

Programming

Since almost all modern electronic systems contain programmable devices it is essential that all candidates are able to analyse a process or operation into a sequence of fundamental operations, and then represent these fundamental operations into a diagrammatic form.  While there are several diagrammatic systems available, this specification continues to use flowcharts for this purpose.

Within the foreseeable future, microcontrollers will all continue to have various forms of Assembler Language, even though high level interpreters are usually available.  It is not be possible to cater for all of the high level interpreters within an examination specification and so it was decided to focus on the fundamental concepts of microcontrollers and develop skills that are transferable to future devices.  As a result, this specification requires candidates to be familiar with a limited range of assembler language microcontroller instructions.  These instructions are listed below and will also be available on the Data sheet included with the examination paper:-

- the memory is made up of registers, each with its own separate location, R;

- K is used to represent a literal, which can be a memory location (e.g. 29h), a label (e.g. display) or a value, (e.g. FAh);

- standard arithmetic and Boolean operators, e.g. add K to W etc as in the table below;

| Mnemonic | Operands | Description | Operation | Flags | Clock cycles |
| --- | --- | --- | --- | --- | --- |
| NOP | none | No operation | none | none | 1 |
| CALL | K | Call Subroutine | stack <=PC PC <= K | none | 2 |
| RET | none | Return from Subroutine | PC <= stack | none | 2 |
| INC | R | Increments the contents of R | (R) <= (R) + 1 | Z | 1 |
| DEC | R | Decrements the contents of R | (R) <= (R) - 1 | Z | 1 |
| ADDW | K | Add K to W | W <= W + K | Z, C | 1 |
| ANDW | K | AND K with W | W <= W • K | Z, C | 1 |
| SUBW | K | Subtract K from W | W <= W - K | Z, C | 1 |
| ORW | K | OR K and W | W <= W + K | Z, C | 1 |
| XORW | K | XOR K and W | W <= W  K | Z, C | 1 |
| JMP | K | Jump to K (GOTO) | PC <= K | none | 2 |
| JPZ | K | Jump to K on zero | PC <= K if Z=1 | Z=1 | 2 |
| JPC | K | Jump to K on carry | PC <= K if C=1 | C=1 | 2 |
| MOVWR | R | Move W to the contents of R | (R) <= W | Z | 1 |
| MOVW | K | Move K to W | W <= K | Z | 1 |
| MOVRW | R | Move the contents of R to W | W <= (R) | Z | 1 |

Candidates will not be expected to remember the details of this generic microcontroller as sufficient information will always be included within the Data Sheet and the questions.  However, they will need to understand the concepts and the application of such instructions to microcontrollers.

Candidates will be expected to be familiar with the following representations of hexadecimal numbers:-    FA16,    FAh,    &HFA,    0xFA

In general candidates will be required to either write a subroutine for one of the operations listed within the specification or to interpret a piece of code written with this instruction set.

Candidates are also expected to be aware of both the use of hardware interrupts and polling of input ports to trigger events, and should be able to consider the advantages and disadvantages of each method.

Input subsystems

This section is concerned with devices which can be used to enter information into a microcontroller system.  There has only been one addition to this section from the previous specification, and candidates are now required to be able to explain the benefits of using a Gray coded shaft encoder compared to a binary coded shaft encoder.

Output subsystems

This section is concerned with devices which can receive information from a microcontroller system.  It has been decided to continue to use a summing amp based DAC within this specification rather than move to R - 2R type devices.  The use of the summing amplifier follows on directly from the AS module work of ELEC2.

Candidates should know about multiplexed seven segment displays and their advantages over non-multiplexed displays.  Candidates should be aware of the relative merits of both LED and LCD seven segment displays as well as the overall limitations of such displays for showing a wide variety of different characters.  Candidates should be aware how these limitations can be overcome by the use of multiplexed dot matrix displays.

Most microcontroller operated machines require motors as an output device.  This specification requires candidates to be aware that there are two main types of motor, conventional and stepper and they should be able to describe the essential differences in operation between these two types of motors.

Candidates are required to be able to describe the use and operation of stepper motors and be aware that there are both bipolar and unipolar stepper motors and these can each be 4 pole or 6 pole devices.

Interfacing subsystems

This section is similar to ELE4 in the previous specification.  The Schmitt trigger has been restricted to inverting circuits and, as before, these should be based on op-amps.

The control circuits needed to drive both LED and LCD seven segment multiplexed displays and LED dot matrix displays should be studied by the candidates.  It is expected that examination questions involving these displays will often show these displays driven from the output ports of a microcontroller.

Candidates will be expected to be able to describe the associated circuits needed to drive both conventional and stepper motors, though again it is expected that, apart from MOSFET switches, this will be accomplished from the output ports of a microcontroller.

New to this section is the H-bridge driver circuit for conventional motors.  Candidates are expected to be able to recall the circuit and describe its operation.

Robotic systems

Recent years have seen a dramatic growth in the use of robotic systems, particularly in industrial and military applications.  While there are the headline catching robots like the Mars Rover and cruise missiles, the vast majority are simply systems which may or may not be mobile but which are able to have some awareness of its surroundings and, through training, can act upon that information.  This growth in the use of robotic systems is set to increase as technology increases.

This section of the specification gives candidates an opportunity to gain a general awareness of the diverse use of robotic systems as well as likely future developments.  It also provides a focus for this module on programmable control systems.

Candidates should be able to describe the essential components of robotic systems.  All robotic systems need sensors.  These can range from simple switches which are actuated when the robot touches something, through ultra sound or microwave echo location to full video recognition.  Candidates will not be expected to have any depth of knowledge of these sensors, but be aware of some possible types and their uses.

In order for a robotic system to interact with its surroundings, it must have actuators that it can use to move, grasp etc.  Candidates will have already studied both stepper and conventional motors as part of this specification but they should also be aware that robotic systems can also use both hydraulic and pneumatic actuators.  Again candidates will not be expected to have any depth of knowledge of these actuators, but be aware of some possible types, their applications advantages and disadvantages.

The information arriving from the sensors will need processing, and instructions will need to be sent to the actuators.  This is the job of the control system.  In most cases this will take the form of a microprocessor control system which candidates will have already studied as part of this specification.

All robotic systems need sources of power.  For fixed systems this does not represent a problem since it is easy to provide all the power required from the mains electricity supply.  However, for robotic systems that are mobile, the source of power can be a major issue.  Candidates should be aware that all mobile robotic systems need some way of storing energy, which in most cases will be batteries.  Candidates should be aware of at least two different types of rechargeable battery, e.g. Lead acid, Nickel Metal Hydride, Lithium Ion etc and the relative merits of each type.  Candidates should also be aware of the increasing use of fuel cells, particularly those based on alcohol, and the advantages and disadvantages of these compared to conventional rechargeable batteries.  Again candidates will not be expected to have any great depth of knowledge of these power sources, but be aware of some possible types and their relative merits.

Candidates are required to be able to analyse a process into its fundamental operations and write subroutines to perform basic tasks.  It is envisaged that many of these processes will relate to robotic systems and so there is little additional material to cover for the section on designing control algorithms for a robotic system to achieve a given objective.  However, candidates will benefit from having given some prior consideration to such examples as:-  a robot, moving forward, receives information from its sensors that there is an obstacle ahead.

Any autonomous robotic system will have been programmed/trained to deal with a wide variety of situations it is likely to encounter.  However, it is inevitable that it will encounter situations that it has not previously met.  In order to be able to learn, the techniques used within Artificial Neural Networks, e.g. Back Propagation are likely to be employed.  Candidates should be aware of at least one of these ANN techniques and should also develop a reasoned opinion as to the ability of such systems to sustain artificial intelligent behaviour.

Candidates will need to be aware of suitable applications of robotic systems as well as the social and economic impact of such systems.  Clearly, with the technological developments of each passing year, the capability of robotic systems will continue to develop and candidates should develop a reasoned opinion of possible future developments.