NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

Control

Systems;

general

principles

PACKAGE

(1).	Characteristics of Open and Closed loop control systems.

(2).	Feedback in Control Systems.

(3).	Positive and Negative feedback in control systems

(4).	General features

A).(1).		Open and Closed Loop Control Systems

The aim of this assignment is to develop an understanding of the difference between Open and Closed loop control systems.

1).	a).	Explain what is meant by an Open Loop Control System.

.............................................................................................................................

.............................................................................................................................

b).	Draw a labelled block diagram of a general Open Loop control system.

c)	Describe an example of an Open Loop control system.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

d).	Explain how a Closed Loop control system differs from an Open Loop system.

.............................................................................................................................

.............................................................................................................................

e).	Draw a labelled block diagram of a general Closed Loop control system.

f).	Describe an example of a Closed Loop control system

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

A).(2).		Feedback.

The aim of this assignment is to develop an understanding of what is meant by Feedback in control systems and to identify systems that use feedback.

1).	a).	Explain what is meant by Feedback.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

b).	Explain what is meant by Negative Feedback.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

c).	Describe an example of a control system that uses Negative Feedback.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

d).	Explain what is meant by Positive Feedback.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

e).	Describe an example of a control system that uses Positive Feedback.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

2).	a).	Explain what is meant by 'hunting' in a control system.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

b).	Describe an example of a control system where hunting can be a nuisance.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

c).	Hunting can be overcome by introducing Hysteresis.  Explain what is meant by 			Hysteresis.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

d).	Suggest an advantage and a disadvantage of having a control system with hysteresis.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

e).	Suggest a situation where neither Hunting nor Hysteresis would be acceptable.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

f).	Suggest alternative methods of control to minimise Hunting.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

A).(3).		Electronic control circuits.

The aim of this assignment is to develop an understanding of how an electronic control system can be implemented with electronic circuits.  This will help relate the concepts in the A2 course to those of the AS course.  This will be useful for Synoptic questions.

1).	The diagram below shows a water tank.  The water valve operates from 12V and is open when current passes through it.  The water level sensor has a low resistance when wet and a high resistance when dry.

a).	Draw a circuit diagram for the control system in the space below.

(The alpha circuit boards are useful for making a model of this.)

b).	Explain how the circuit works.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

2).	The diagram below shows a water tank.  The water valve operates from 12V and is open when current passes through it.  There are two water level sensors, one for the high level and one for the low level.  Each water sensor has a low resistance when wet and a high resistance when dry.  The water valve is to turn on when the low water level sensor is dry and turn off when the high water level sensor is wet.

a).	Draw a circuit diagram for the control system in the space below.

(The alpha circuit boards are useful for making a model of this.)

b).	Explain how the circuit operates.

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

.............................................................................................................................

A).(4).		Research Assignment.

Examine your home for Control Systems (they are everywhere on modern consumer goods) and select six that you will investigate.  You can choose some mechanical systems  if you wish.

For each of the chosen control systems, identify whether it is open or closed and whether it has positive or negative feedback.

Also identify, where possible, the source of input instruction(s), the processor, the driver, the output device and any feedback.

Prepare a brief report on these control systems and be prepared to talk to the group about your findings.