NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

AUDIO

SYSTEMS

PACKAGE

(1).	Capacitative reactance

(2)	RC low-pass passive filters

(3).	RC high-pass passive filters

(4).	First order active filters

(5)	Audio Power Amplifier ICs

B).(1).		Capacitative reactance.

The aim of this assignment is to investigate and understand capacitative reactance.

You will need to use the Support Booklet.

1).	This first experiment is to investigate how the alternating current through a capacitor 		varies with the alternative voltage applied across it, with the frequency constant.

a).	Set up the circuit shown below using connecting leads and crocodile clips.

Use the circuit to investigate how the alternating current passing through the capacitor is 	dependent on the alternating voltage across the capacitor.  Complete the table below with 	your measurements.

| Alternating Voltage (V) | Alternating Current (mA) | V/I (volts/amps) ohms |
| --- | --- | --- |
| 0 |  |  |
| 0.2 |  |  |
| 0.4 |  |  |
| 0.6 |  |  |
| 0.8 |  |  |
| 1.0 |  |  |
| 1.2 |  |  |
| 1.4 |  |  |
| 1.6 |  |  |
| 1.8 |  |  |
| 2.0 |  |  |

(b)	Calculate V/I for each row, remembering to change the current into Amps.

What do you notice about each of the values?

..................................................................................................................................

How does this compare with the Ohm's Law experiment in the Foundation Module?

..................................................................................................................................

2).	The next experiment is to investigate how the alternating current through a capacitor is 	affected by the frequency of the supply.

a).	Set up the circuit shown below using connecting leads and crocodile clips.

Use the circuit to investigate how the alternating current passing through the capacitor is 	dependent on the frequency of the supply.  You will need to adjust the output voltage to 	2.5V each time you adjust the frequency.

Complete the table below with your measurements.

| Frequency (Hz) | Alternating current (mA) | V/I (2.5/I (A)) ohms |
| --- | --- | --- |
| 100 |  |  |
| 200 |  |  |
| 400 |  |  |
| 600 |  |  |
| 800 |  |  |
| 1000 |  |  |
| 2000 |  |  |
| 4000 |  |  |
| 6000 |  |  |
| 8000 |  |  |
| 10000 |  |  |

(b)	Calculate V/I for each row, remembering to change the current into Amps and that 		the voltage is 2.5V.

What do you notice about each of the values?

..................................................................................................................................

How does V/I vary with frequency?

..................................................................................................................................

..................................................................................................................................

(c)	What is the name for V/I for a capacitor and state its units?

..................................................................................................................................

(d)	The symbol for this quantity is Xc.

State the formula that relates Xc to frequency and capacitance.

..................................................................................................................................

..................................................................................................................................

(e)	On the axes below draw a sketch graph to show how Xc varies with frequency.

Xc

frequency

(f)	On the axes below draw a sketch graph to show how I varies with frequency.

frequency

current

(g)	Complete the summary below.

Xc is known as the ................. of a capacitor.  It is the ratio of the alternating

.................. divided by the alternating ...............  Xc is measured in ...............

Xc is also dependent upon the                   of the supply.  As the frequency 	increases, Xc ........................  The formula for Xc is

Xc =

B).(2).		RC passive low-pass filters.

The aim of this assignment is to understand how low pass filters can be made from a resistor and capacitor and to be able to predict the 'break point' or "cut off frequency" for the circuit.  You will need to use the Support Booklet.

1).	This investigation is to find how the ratio of the output voltage to the input voltage varies 	with frequency for a first order low pass filter.

a).	Set up the circuit shown below using a protoboard, connecting leads and crocodile 		clips.

Use the circuit to investigate how the output voltage varies with frequency if the input 	voltage is kept constant.  You will need to adjust the output voltage to 2.5V each time 	you adjust the frequency.

Complete the table below with your measurements.

| Frequency (Hz) | Output voltage (V) | Vout/Vin (Vout/2.5) |
| --- | --- | --- |
| 100 |  |  |
| 200 |  |  |
| 400 |  |  |
| 600 |  |  |
| 800 |  |  |
| 1000 |  |  |
| 2000 |  |  |
| 4000 |  |  |
| 6000 |  |  |
| 8000 |  |  |
| 10000 |  |  |
| 20000 |  |  |
| 40000 |  |  |
| 60000 |  |  |
| 80000 |  |  |
| 100000 |  |  |

(b)	What do you notice about how Vout/Vin changes with frequency?

..................................................................................................................................

(c)	On the log-log graph axes below, plot the results of your investigation.

(d)	Use your graph to find the frequency at which the Vout/Vin is equal to 0.71.

..................................................................................................................................

(e)	What is the name and significance of this frequency?

..................................................................................................................................

..................................................................................................................................

(f)	Check that your value is in agreement with the value given by the formula.

fo=12πRC

..................................................................................................................................

..................................................................................................................................

(g)	What is the name given to this formula?

..................................................................................................................................

B).(3).		RC passive high-pass filters.

The aim of this assignment is to understand how high pass filters can be made from a resistor and capacitor and to be able to predict the 'break point' or "cut off frequency" for the circuit.  You will need to use the Support Booklet.

1).	This investigation is to find how the ratio of the output voltage to the input voltage varies 	with frequency for a first order low pass filter.

a).	Set up the circuit shown below using a protoboard, connecting leads and crocodile 		clips.

Use the circuit to investigate how the output voltage varies with frequency if the input 	voltage is kept constant.  You will need to adjust the output voltage to 2.5V each time 	you adjust the frequency.

Complete the table below with your measurements.

| Frequency (Hz) | Output voltage (V) | Vout/Vin (Vout/2.5) |
| --- | --- | --- |
| 100 |  |  |
| 200 |  |  |
| 400 |  |  |
| 600 |  |  |
| 800 |  |  |
| 1000 |  |  |
| 2000 |  |  |
| 4000 |  |  |
| 6000 |  |  |
| 8000 |  |  |
| 10000 |  |  |
| 20000 |  |  |
| 40000 |  |  |
| 60000 |  |  |
| 80000 |  |  |
| 100000 |  |  |

(b)	What do you notice about how Vout/Vin changes with frequency?

..................................................................................................................................

(c)	On the log-log graph axes below, plot the results of your investigation.

(d)	Use your graph to find the frequency at which the Vout/Vin is equal to 0.71.

..................................................................................................................................

(e)	What is the name and significance of this frequency?

..................................................................................................................................

..................................................................................................................................

(f)	Check that your value is in agreement with the value given by the formula.

fo=12πRC

..................................................................................................................................

..................................................................................................................................

(g)	What is the name given to this formula?

..................................................................................................................................

B).(4).		First order active filters.

The aim of this assignment is to understand how passive filters can be used with op-amps to create active first order filters.  You will need to use the Support Booklet.

1).	Suggest an advantage of using op-amps with passive RC filters.

..................................................................................................................................

..................................................................................................................................

2).	Examine the circuit diagrams below.  If you are having difficulties predicting the frequency 	response of the circuit, and if there is time, construct the circuit and investigate the circuit 	practically.

(a)

(i)	What type of filter circuit is this?

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at high frequencies?

..................................................................................................................................

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at low frequencies?

..................................................................................................................................

..................................................................................................................................

(iii)	What is the cut-off frequency for the amplifier?

..................................................................................................................................

..................................................................................................................................

(b)

(i)	What type of filter circuit is this?

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at high frequencies?

..................................................................................................................................

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at low frequencies?

..................................................................................................................................

..................................................................................................................................

(iii)	What is the cut-off frequency for the amplifier?

..................................................................................................................................

..................................................................................................................................

(iv)	On the axes below sketch the frequency response?

(c)

(i)	What type of filter circuit is this?

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at high frequencies?

..................................................................................................................................

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at low frequencies?

..................................................................................................................................

..................................................................................................................................

(iii)	What is the cut-off frequency for the amplifier?

..................................................................................................................................

..................................................................................................................................

(iv)	On the axes below sketch the frequency response?

(d)

(i)	What type of filter circuit is this?

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at high frequencies?

..................................................................................................................................

..................................................................................................................................

(ii)	What is the voltage gain of the amplifier at low frequencies?

..................................................................................................................................

..................................................................................................................................

(iii)	What is the cut-off frequency for the amplifier?

..................................................................................................................................

..................................................................................................................................

(iv)	On the axes below sketch the frequency response?

B).(5).		Audio Power Amplifier ICs.

The aim of this assignment is to understand the properties and how to use audio power amplifier ICs.  You will need to use the Support Booklet.

1).	(a)	Explain what is meant by the term voltage gain.

.............................................................................................................................................

.............................................................................................................................................

(b)	A power amplifier is required to produce an output of 8Vrms across an 8Ω 			loudspeaker from an input voltage of 0.05Vrms.  Calculate the voltage gain of the 			amplifier?

.............................................................................................................................................

.............................................................................................................................................

2).	(a)	Explain what is meant by the term power gain.

.............................................................................................................................................

.............................................................................................................................................

(b)	A power amplifier is required to produce an output of 20W across an 8Ω 			loudspeaker from an input power of 5mW.  Calculate the power gain of the 			amplifier?

.............................................................................................................................................

.............................................................................................................................................

3).	(a)	Explain what is meant by the term bandwidth for an amplifier, both in terms of 			power and voltage gain.

.............................................................................................................................................

.............................................................................................................................................

.............................................................................................................................................

.............................................................................................................................................

.............................................................................................................................................

(b)	Explain why the power gain of an amplifier decreases both at high and low 			frequencies.

.............................................................................................................................................

.............................................................................................................................................

.............................................................................................................................................

(c)	What is the bandwidth of the amplifier whose frequency response is shown below?

.............................................................................................................................................

(d)	What is the bandwidth of the amplifier whose frequency response is shown below?

4).	Summarise the advantages and disadvantages of using IC audio amplifiers instead of 	discrete component audio amplifiers.

.............................................................................................................................................

.............................................................................................................................................

.............................................................................................................................................

Predicted results

I).(1)(1)(a)

| "Freq (Hz)" | "Current (mA)" | "V/I (ohms)" |  | "Voltage (V)" | "Current (mA)" | "V/I (ohms)" |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | 0.16 | 15625 |  | 0 | 0 | #DIV/0! |
| 200 | 0.31 | 8065 |  | 0.2 | 1.26 | 158.7 |
| 400 | 0.63 | 3968 |  | 0.4 | 2.51 | 159.4 |
| 600 | 0.94 | 2660 |  | 0.6 | 3.77 | 159.2 |
| 800 | 1.26 | 1984 |  | 0.8 | 5.03 | 159 |
| 1000 | 1.57 | 1592 |  | 1 | 6.28 | 159.2 |
| 2000 | 3.14 | 796 |  | 1.2 | 7.54 | 159.2 |
| 4000 | 6.28 | 398 |  | 1.4 | 8.8 | 159.1 |
| 6000 | 9.42 | 265 |  | 1.6 | 10.05 | 159.2 |
| 8000 | 12.57 | 199 |  | 1.8 | 11.31 | 159.2 |
| 10000 | 15.71 | 159 |  | 2 | 12.57 | 159.1 |
| "V=2.5, C=100nF" |  |  |  | "f=10kHz,C=100nF" |  |  |

| "Freq (Hz)" | "Vout (V)" | "Vout/2.5" |  | "Vout (V)" | "Vout/2.5" |
| --- | --- | --- | --- | --- | --- |
| 100 | 2.5 | 1 |  | 0.02 | 0.01 |
| 200 | 2.48 | 0.99 |  | 0.03 | 0.01 |
| 400 | 2.42 | 0.97 |  | 0.06 | 0.03 |
| 600 | 2.34 | 0.94 |  | 0.09 | 0.04 |
| 800 | 2.23 | 0.89 |  | 0.13 | 0.05 |
| 1000 | 2.12 | 0.85 |  | 0.16 | 0.06 |
| 2000 | 1.56 | 0.62 |  | 0.31 | 0.12 |
| 4000 | 0.92 | 0.37 |  | 0.61 | 0.24 |
| 6000 | 0.64 | 0.26 |  | 0.88 | 0.35 |
| 8000 | 0.49 | 0.2 |  | 1.12 | 0.45 |
| 10000 | 0.39 | 0.16 |  | 1.33 | 0.53 |
| 20000 | 0.2 | 0.08 |  | 1.96 | 0.78 |
| 40000 | 0.1 | 0.04 |  | 2.32 | 0.93 |
| 60000 | 0.07 | 0.03 |  | 2.42 | 0.97 |
| 80000 | 0.05 | 0.02 |  | 2.45 | 0.98 |
| 100000 | 0.04 | 0.02 |  | 2.47 | 0.99 |
| "Vin =2.5V, R = 1k, C = 100nF" |  |  |  | "Vin =2.5V, R = 1k, C = 10nF" |  |
| "Results for low pass" |  |  |  | "Results for high pass" |  |