NAME: …………………………

Core practical 11

Display and analyse the potential difference

across a capacitor as it discharges through a resistor.

Objective:

To measure the time constant for the discharge

To observe the effect of changing the value of the resistor

CPAC skills:-	2b,				3a,				5b

Safety

You are assessed against CPAC 3a and should produce an appropriate risk assessment.

Do not exceed the voltage rating of the capacitor.

All open circuit work should be carried out below 40 V DC.

All the maths you need

Recognise and make use of appropriate units in calculations.

Recognise and use expressions in decimal and standard form.

Use ratios, fractions and percentages.

Use calculators to find and use power, exponential and logarithmic functions.

Use an appropriate number of significant figures.

Identify uncertainties in measurements and use simple techniques to determine uncertainty
	when data are combined by addition, subtraction, multiplication, division and raising to
	powers.

Substitute numerical values into algebraic equations using appropriate units for physical
	quantities.

Use logarithms in relation to quantities that range over several orders of magnitude.

Translate information between graphical, numerical and algebraic forms.

Plot two variables from experimental or other data.

Understand that y = mx + c represents a linear relationship.

Determine the slope and intercept of a linear graph.

Interpret logarithmic plots.

Use logarithmic plots to test exponential and power law variations

Equipment

Pasco GLX Explorer data logger with voltage input probe.

Electrolytic capacitor selection box - select 1000F

Resistor selection box. - select 47k

Variable power supply - to give 5V.

Multimeter - set to measure resistance.

Connection leads.

Procedure

1.	Use the multimeter to measure the value of the selected resistor R and record your result.

2.	Familiarise yourself with using the data loggers, in conjunction with reading the data logger 	instruction sheets

3.	Set the data loggers to record voltage every 10s.

4.	Construct the circuit shown in the diagram overleaf - check polarity of devices.

5.	Move the flying lead so that the capacitor C charges up.

6.	Move the flying lead, so that the capacitor begins to discharge and at the same time start
	the data logger recording measurements.

7.	Take sufficient readings to enable you to plot a graph of the results and calculate the time

constant for the discharge.

8.	Record your measurements from the data logger in a suitable table and add an extra column 	to show the value of Ln(V).

Analysis of results

1.	Plot a suitable graph of your readings in order to determine a value for the time constant
	of the discharge.

2.	Use your value for the time constant to calculate a value for the capacitance and compare
	your value with the stated value of the capacitor.
	Consider your uncertainty to conclude whether the capacitor is within its tolerance.

(Assume that time and voltage measurements have an uncertainty of 2% and resistance
	measurement has an uncertainty of 1%).

Learning tips

When using unfamiliar equipment it is essential to make various to familiarise yourself
	with the device.  Do this before using it to make measurements.

Since the rate of discharge depends on how much charge is left, the discharge is exponential
	according to the formula

V=V0e−tRC

Taking logs to the base e of each side gives Ln(V) = Ln(V0) -t/RC

This is similar to y = mx + c  The value of the gradient will be -1/RC

The tolerance on the value of the capacitor – often 20% – is not a measurement uncertainty.

Questions

1.	Draw the circuit that you could use to measure the time constant as the capacitor is charged.

2.	Explain why using data loggers is likely to reduce your uncertainties.

Using the GLX Explorer

Switch On

Connect the mains power supply.

Press the On Button

Switch Off

Press the On Button and hold it down until the data logger switches off.

Screen contrast.

While pressing the Home key, use the up and down cursor buttons.

Setting the sample rate to 10s

Press the Home key to return to the Home screen.

Press the key F4 which opens the sensor screen.

Sample Rate Units is highlighted.

Press  and select  seconds  from the drop down menu using the
		 and  keys and then press .

Press the  key to highlight Sample Rate and press .

Select 10 from the drop down menu using the  and  keys and then press .

Press the Home key.

Deleting a Data File.

Press the Home key to return to the Home screen.

Select the Data Files screen using the  key.

Select the file to delete using the cursor keys (if necessary).

Press F3 and then F1 to delete the file.

Press the Home key to return to the Home screen.

Taking Measurements

1	Use the     and  keys to highlight Table.

Press the  key and a table of results should be displayed.

2).	Press the           button to start while disconnecting the flying from the power supply.

3).	When you have taken sufficient samples press the           button again to stop the samples.