GLOSSARY OF PRACTICAL TERMS.

| Accuracy | A measured result is accurate if it is close to the true value. |
| --- | --- |
| Calibration | Marking a scale on a measuring instrument. |
| Data | Information that has been collected. |
| Errors | Also see Uncertainties. |
| - measurement error | The difference between a measured value and the true value. |
| - anomalies | Values in a set of results which are judged not to be part of the variation caused by random uncertainty. |
| - non-linearity error | This is when the instrument response does not vary directly as the property. It can be detected by using three point calibration. |
| - proportional error | This is when the instrument consistently reads a constant percentage too high or too low. This is likely to be due to an amplifier error or poor calibration. |
| - random error | These cause readings to be spread about the true value, due to results varying in an unpredictable way from one measurement to the next. They are present when any measurement is made and cannot be corrected. The effect of random errors can be reduced by making more measurements and calculating a mean. |
| -systematic error | These cause readings to differ from the true value by a consistent amount each time a measurement is made. They cannot be corrected with simple repeats. If a systematic error is suspected then the data collection should be repeated using a different technique or different measuring equipment and the results compared. |
| - zero error (offset error) | When a measuring instrument does not read zero under the conditions of no input. |
| Evidence | Data which has been shown to be valid |
| Fair test | This is when only the independent variable has been allowed to affect the dependent variable. |
| Hypothesis | A proposal intended to explain certain facts or observations. |
| Interval | The quantity between readings. |
| Precision | Precise measurements are when there is very little spread about their mean value. Precision depends only on the extent of random errors – it gives no indication of how close the results are to the true value. |

| Prediction | A statement suggesting what will happen in the future based on observation, experience or a hypothesis. |
| --- | --- |
| Range | The difference between the maximum and minimum values of a variable. |
| Repeatable | A measurement is repeatable if the original experimenter repeats the investigation using the same method and equipment and obtains the same results. |
| Reproducible | A measurement is reproducible if the investigation is repeated by another person, or by using different equipment or techniques and the same results are obtained. |
| Resolution | The smallest change in the quantity being measured of a measuring instrument that gives a perceptible change in the reading. |
| Sketch graph | A line graph that shows the general shape of the relationship between two variables. |
| True value | The value obtained in an ideal measurement. |
| Uncertainty | The interval within which the true value can be expected to lie e.g. 20ºC ± 2ºC |
| Validity | The suitability of the investigative procedure to answer the question being asked. E.g. the procedure would not be valid if it were not a fair test. |
| Valid conclusion | A conclusion supported by valid data, obtained from an appropriate experimental design and based on sound reasoning |
| Variables | These are physical, chemical or biological quantities of characteristics. |
| - categoric | Categoric variables have values that are labels, e.g. names of plants |
| - continuous | Continuous variables can have values (called a quantity) that can be given a magnitude either by counting (e.g. the number of birds) or by measurement (e.g. light intensity). |
| - control | A control variable is one which may, in addition to the independent variable, affect the outcome of the investigation and therefore has to be kept constant or at least monitored. |
| - dependent | The dependent variable is the variable of which the value is measured for each and every change in the independent variable. |
| - independent | The independent variable is the variable for which values are changed or selected by the investigator. |