NAME: …………………………

Core practical 3

Determine the emf and internal resistance of an electrical cell.

Objective

To make measurements using an electrical circuit.

CPAC skills:-	2a,				2d,				5a

Safety

Voltages and currents are small and so present no specific hazard.

Normal laboratory safety procedures should be followed.

All the maths you need

The mathematical model for this circuit is E  – Ir = V.

Recognise and make use of appropriate units in calculations.

Use an appropriate number of significant figures.

Identify uncertainties in measurements and use simple techniques to determine uncertainty
	when data are combined by addition, subtraction, multiplication, division and raising to
	powers.

Translate information between graphical, numerical and algebraic forms.

Plot two variables from experimental or other data.

Understand that y = mx + c represents a linear relationship.

Determine the slope and intercept of a linear graph.

Equipment

A mounted electrical cell with an added internal resistance, labelled r in diagram.

A 1Ω to 150Ω resistance box, labelled R in the diagram

A voltmeter on 2 V range

An ammeter on 200 mA range

2 crocodile clips to connect to the mounted cell

5  4mm plug leads to make electrical connections.

Diagram

Procedure

1.	Connect your equipment as in the circuit diagram above.

As a plan for this practical, draw a labelled circuit that will connect the resistance box
	across 	the mounted cell and measure the potential difference, V, across R and
	the current, I through it.

2.	Have your circuit layout checked before you begin.

3.	Comment on how you will ensure the equipment is safely used and continues to
	function properly.

4.	Choose different resistances on the resistor box and record values for V and I.
	Take readings for a suitable range of different resistances.

5.	Put your measurements of V and I into a suitable table.

Analysis of results

1.	The mathematical model for this circuit is E – Ir= V.

2.	So V= –rI + E.  A graph of V against I should give a straight line with gradient –r.

3.	Measure the gradient of your graph and compare it with the manufacturer’s value of the 
	resistor, r.
	Ensure you take account of the powers of 10 (prefix) in the measurement of current.

4.	Justify the number of significant figures you use in your answer.

5.	Comment on the likely accuracy of your values for E and r.

Learning tips

It is not usually necessary to repeat readings from an electrical circuit such as this.
	There is very little random error and very little judgement is required when taking readings.
	For this reason, you should take more than the usual six readings so that you have more
	points on your graph.  This will make it easier to recognise anomalies.

Questions

1.	When the internal resistance is large in comparison to the external resistance, the terminal
	potential difference falls to a small value.  This can be used to make high voltage supplies
	safe for use in a laboratory.  Explain how this makes the supply safe.

2.	It should not matter whether the voltmeter is connected across R or across the cell.
	This is partly because of the low resistance of the ammeter.  Explain why.

3.	The intercept of your graph will be very close to the true value for the emf of the cell.
	Account for any difference.

4.	Explain any difference between your value for r and the manufacturer’s value.