NAME: …………………………

Core practical 4

Use a falling-ball method to determine the viscosity of a liquid

Objective:

To time the fall of a ball through washing-up liquid to determine the viscosity.

CPAC skills:-	1a,				2b,				4b

Safety

Washing-up liquid spills are very slippery and must be cleared up at once.
	(Have paper towels to hand.)

Ball bearings are a major slip hazard, and any dropped on the floor must be recovered.

Normal laboratory safety procedures should be followed.

All the maths you need

Mass = density × volume

At terminal velocity, the vector sum of the forces on the ball is zero.

weight – drag – upthrust = 0

From this it can be shown that   η= 2r2g(ρ – σ)

9v

where η is the viscosity of the liquid, r is the radius of the ball,
		ρ is the density of the ball, σ is the density of the liquid and
		v is the terminal velocity.

The percentage uncertainty in r2 is double the percentage uncertainty in r.
	The percentage uncertainties in the densities should be small enough to ignore.

Recognise and make use of appropriate units in calculations.

Recognise and use expressions in decimal and standard form.

Use ratios, fractions and percentages.

Use an appropriate number of significant figures.

Find arithmetic means, identify uncertainties in measurements and use simple techniques to

determine uncertainty when data are combined by addition, subtraction, multiplication, 
	division and raising to powers.

Substitute numerical values into algebraic equations using appropriate units for physical

quantities.

Equipment

250ml measuring cylinder filled with washing up liquid.

Ball bearings of 1.5mm and 3mm diameter.

stop clock or timer.

rubber bands to mark distances.

30cm or metre ruler.

micrometer screw gauge.

Balance.

1 × large ball bearing.

NOTE.  250cm3 of washing up liquid has a mass of 252g.

Procedure

1.	Weigh the large ball bearing, measure its radius, r, and hence calculate the density, ρ, of the
	ball bearing material.

2.	Place two rubber bands around the tube.  The highest should be far enough below the 
	surface of the liquid to ensure the ball is travelling at terminal velocity when it reaches this
	band. (230ml mark).  This is where the timer is started.
	The remaining band should be far enough apart to allow a reasonable time intervals to be
	measured. (30ml mark).
	This will enable you to measure the terminal velocity or each falling ball.

3.	Start the timer when the ball passes the highest rubber band and stop the timer when the ball
	passes the lower rubber band.
	Adjust the position of the rubber bands if your first test is not suitable.

4.	Once you are happy with the position of the rubber bands, measure the distance, d, between

them.

5.	Repeat at least three times for balls of this diameter and three times for each different
	diameter.

6.	Record all of your measurements appropriately.

Analysis of results

1.	For each diameter, calculate mean values for the time.

2.	Use these mean values and the distances to calculate mean values for the terminal velocity
	of each ball.  Repeat for all the balls of the same diameter to obtain a mean value.

3.	By considering the spread in your repeated readings, estimate the uncertainty of your mean
	values.  This is usually half of the range.

Learning tips

Position your eyes at the horizontal level of the rubber bands when starting and stopping the

timer.  You need to develop a good technique for measuring the time so that you are 	consistent.

For example, if you read the time as the bottom of the ball crosses the top of the band, you

should always do so.  Make sure you also measure your distances from this point.

Questions

1.	Explain why you would not use light gates to measure the time.

2.	Sometimes the balls fall close to the wall.

Comment on the effect that this will have on the measurements.

3.	Use your uncertainty values to estimate the uncertainty in your value for the viscosity of the

washing-up liquid.