Control Electronics

Revision Questions

from

previous syllabi.

These do not cover the complete specification but may provide useful examination practice.

When considering these questions please remember that approximately 40% of each A2 module examination must consist of synoptic questions.

1).	The circuit diagram for a thermostat for a hot water system is shown below.

The characteristic for the thermistor is shown in the graph below.

(a)	The op-amp is arranged as a Schmitt trigger.  Calculate the values of the input 			voltage, V, which make the output of the op-amp switch from +12 V to -12 V and 			from -12 V to +12 V.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(b)	Calculate the maximum and minimum temperatures of the hot water system.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(c)	Explain which resistor would have to be changed to alter the mean temperature of the 		hot water.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(8)

2).	A hot wire anemometer is an instrument used to measure wind speed.  It consists of a small 	piece of wire heated to a temperature of approximately 300oC, in still air, by passing an 	electric current through it.  As the wind blows air past it the hot wire is cooled and the 	resistance of the wire changes.  The diagram below shows how the resistance of the hot wire 	changes with wind speed.

The hot wire is connected to the circuit shown below

(a)	(i)	Calculate the power dissipated in the hot wire when it is in still air.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(ii)	What would be a suitable power rating for the 20  resistor?

..............................................................................................................................

..............................................................................................................................

(1)

(b)	(i)	Determine the value of the variable resistor so that Vout is zero when the air 			speed is zero.

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	What will be the resistance of the hot wire when the wind speed is 50m/s

..............................................................................................................................

..............................................................................................................................

(1)

(iii)	Calculate Vout when the wind speed is 50m/s.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(5)

(c)	To enable the wind speed to be monitored by a computer, Vout is converted into an 		eight bit number using the circuit shown below

Explain the function of:-

(i)	the Start Conversion input (SC)

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	the End of Conversion output (EoC).

..............................................................................................................................

..............................................................................................................................

(1)

(d)	How often would the wind speed be converted to a digital value by the circuit.

..............................................................................................................................

..............................................................................................................................

(1)

(e)	(i)	Draw a circuit diagram of a NAND gate monostable and explain how it 				functions.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(4)

(ii)	State suitable values of R and C for the monostable to give a time period of 1 			second.

..............................................................................................................................

..............................................................................................................................

(2)

(f)	An output of 111111112 is obtained for an analogue input voltage of 2.55V.

(i)	What wind speed does this correspond to?

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	What is the smallest change in wind speed that could be detected by the 				computer?

..............................................................................................................................

..............................................................................................................................

(1)

(g)	Explain one advantage and one disadvantage of using this system of storing wind 			speed in a computer compared to recording the analogue voltage, Vout, using a chart 		recorder.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(24)

3).	The circuit diagram for a Schmitt trigger is shown below.  Assume that the op-amp 	saturates at + 5V.

(a)	(i)	Calculate the trigger levels of the circuit.

...................................................................................................................

...................................................................................................................

...................................................................................................................

...................................................................................................................

(2)

(ii)	On the axes below sketch a graph to show Vout as Vin varies from –5 V to 			+5 V.

(2)

(b)	A capacitor and resistor are now connected to the Schmitt trigger circuit as shown 			below.

The capacitor is fully discharged and Vout becomes +5 V when the circuit is first 			switched on.

Describe the subsequent changes that occur to the voltage across the capacitor.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(4)

___

(8)

¯¯¯

4).	Instead of producing a detailed instruction booklet, a telephone answering machine 	manufacturer decides to incorporate 'spoken instructions' for the user.

These 'spoken instructions' are stored in a Read Only Memory (ROM) and the whole system 	is controlled by a microcontroller.  A block diagram of the audio output part of the system is 	shown.

(a)	The D/A converter has an 8-bit data input.  How many different analogue voltages 		can be obtained from the output?

...............................................................................................................................

(1)

(b)	Explain the function of the  input on the D/A converter.

...............................................................................................................................

...............................................................................................................................

(2)

(c)	The circuit diagram for a 4-bit D/A converter is shown below.

Each of the digital outputs from the latch is either at +3V for logic 1 or 0V for
		logic 0. What is the value of Vout when the input is 1011?

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

(d)	Explain what changes would have to be made to the circuit above to make it into an 		8-bit D/A converter. Give the values of any other components that you would add.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

(e)	There is inadequate power from the D/A converter to operate a loudspeaker, so it is 		connected to the amplifier circuit shown below.

(i)	Using a diagram, explain what is meant by cross-over distortion.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(2)

(ii)	Explain how D1, D2 and VR1 minimise this effect.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(1)

(iii)	Calculate the voltage gain of the circuit.

...............................................................................................................................

...............................................................................................................................

(1)

(iv)	Estimate the maximum power output to the speaker.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

(f)	To store incoming messages the answering machine contains a 256Kbyte RAM.			The IC has 8 data lines.

(i)	How many memory address inputs does it have?

...............................................................................................................................

(1)

(ii)	If the message is sampled 4000 times per second, what is the maximum 				duration of the message that can be stored?

...............................................................................................................................

...............................................................................................................................

(2)

(g)	By referring to:-

the address bus,

the  input and

the  input

explain how a byte of data is stored in the RAM.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(5)

___

(24)

¯¯¯

5).	The diagram shows part of the block diagram of an environmental monitoring system.

(a)	The input amplifier circuit, shown below, has three input ranges with the most 			sensitive, position A, being 0 - 250 mV. The output of the amplifier is 0 - 1 V and is 		connected to the A/D converter.

(i)	What is the function of the two diodes?

...............................................................................................................................

...............................................................................................................................

(1)

(ii)	Calculate the value of R.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

(iii)	What is the input range when the switch is set to B and C respectively?

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

(b)	The diagram below shows the sample rate generator which allows the unit to be set 		to make samples every 1s, 10s, or 100s.

(i)	Explain why a crystal oscillator is used rather than a free running oscillator.

...............................................................................................................................

(1)

(ii)	How many flip-flops are needed in the frequency divider to give an output of 			1Hz from the 32768Hz crystal oscillator?

...............................................................................................................................

...............................................................................................................................

(2)

(iii)	Draw a diagram to show how four D-type flip-flops can be connected so as to 			divide the input frequency by 10 and provide a mark to space ratio of 1:1.

(5)

(c)	The system uses an 8-bit A/D converter connected as shown below.

(i)	With reference to the diagram above, describe the sequence of events 				that occur when a measurement is made.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(6)

(ii)	If the A/D converter gives a converted value of 111110102 for an input of 				1.00V, what will be the converted value of an input of 125mV?

...............................................................................................................................

...............................................................................................................................

(1)

(iii)	Estimate the percentage uncertainty in the measured value of 125mV as a 				result of the digital conversion.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(2)

___

(24)

¯¯¯

6).		The diagram shows the output signal from a digital communication device.

(a)	Before the signal can be used it has to be converted into a square wave form.

Explain why a voltage comparator is unsuitable for this application and why a 			Schmitt trigger circuit could solve the problem.

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

(3)

(b)	The signal is fed into the Schmitt trigger circuit shown below.

Calculate a suitable value for the resistor R if the voltage switching levels of the 			Schmitt trigger are to be +0.02 V.

................................................................................................................................

................................................................................................................................

................................................................................................................................

(3)

(c)	The signal shown is fed into the Schmitt trigger.  Mark the output waveform onto the 		axes below

(3)

___

(9)

¯¯¯

7).	This question is about the subsystems used in a digital storage unit which can be added to an 	analogue oscilloscope.

(a)	An ADC (A/D converter) has control lines of Start Conversion , Output Enable 		 and End of Conversion (EoC).

(i)	Describe the sequence of signals on the control lines for the ADC to make a 			conversion.

................................................................................................................................

................................................................................................................................

(2)

(ii)	What is the function of the Output Enable  control line?

................................................................................................................................

(1)

(b)	A RAM IC has control lines of Chip Select , Read/Write  Output 			Enable  	and connections to the address bus and data bus.

(i)	Explain the purpose of the address bus.

................................................................................................................................

................................................................................................................................

(1)

(ii)	Describe the sequence of events for a byte of data to be written to a RAM IC.

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

(4)

(c)	An 8-bit counter is made from D-type flip-flops.  Draw a circuit diagram to show 			how a D-type flip-flop can be connected to operate as part of the counter.

(3)

(d)	The clock oscillator is a NAND gate astable as shown in the circuit diagram below.

Explain how the astable works.

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

(4)

The block diagram for the digital storage unit is shown below.  The Y-input is scaled to 	provide a voltage in the range –1.27V to +1.27V.  This analogue voltage is converted into an 	8-bit binary number by the ADC, the most significant bit when set being used to represent 	negative numbers.  Each measured value is then stored in the RAM.  The timebase is derived 	from an 8-bit counter which is converted to an analogue voltage by DAC2 (D/A converter 2) 	before being passed to the X-input of the oscilloscope.

(e)	The oscilloscope has a 10cm wide screen and a display that is scanned the full width 		of the screen.

(i)	How many display points can be stored by the system?

................................................................................................................................

(1)

(ii)	Estimate, showing your working, how far apart each display point is on the 			screen.

................................................................................................................................

................................................................................................................................

(1)

(iii)	Estimate, showing your working, the frequency of the clock oscillator when 			the system is recording a signal with a timebase speed of 1ms/cm.

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

(4)

(iv)	Estimate the voltage at the input of the ADC that would be represented by the 			binary number output of 11011001.

................................................................................................................................

................................................................................................................................

(2)

(v)	When displaying a very rapidly changing input voltage, the screen can 				sometimes become covered in random white dots. Suggest a reason for this.

................................................................................................................................

................................................................................................................................

(1)

___

(24)

¯¯¯

8).	The engine management unit in a modern car measures the speed of the engine using a 	rotation sensor.  The sensor normally has a logic 0 output (<3 V) and gives a logic 1 pulse 	(>9 V) for every 10º of rotation of the engine.  The output from the rotation sensor is 	connected to the Schmitt trigger circuit shown below.

(a)	Suggest a reason why a Schmitt trigger is needed in this application.

...............................................................................................................................

...............................................................................................................................

(1)

(b)	Show that the upper and lower switching voltages of the Schmitt trigger in the circuit 		above are 8 V and 4 V respectively.

Assume that the op-amp output is either +12 V or 0 V.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

c).	Draw a sketch on the axes below to show how the output voltage varies with the input 	voltage.

(3)

___

(7)

¯¯¯

9).	The circuit diagram for a simple computer controlled power supply is shown below.

(a)	State the sequence of signals necessary for a byte of data to be written to the D/A 			converter.

................................................................................................................................

................................................................................................................................

(2)

(b)	The maximum output voltage from the D/A converter is 2.55 V.

(i)	Show that the smallest increment that the output voltage of the D/A can 				change by is 0.01 V.

................................................................................................................................

................................................................................................................................

(1)

(ii)	What would be the output voltage of the D/A converter if the binary input were

11001000?

................................................................................................................................

................................................................................................................................

(2)

(c)	(i)	What is the voltage gain of op-amp 1?

................................................................................................................................

(1)

(ii)	What is the function of op-amp 2?

................................................................................................................................

(1)

(d)	(i)	Explain how the circuit is able to produce an output voltage, Vout, that is 				controlled by the byte of data stored in the D/A converter.

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

................................................................................................................................

(5)

(ii)	What is the purpose of C1 and C2?

................................................................................................................................

................................................................................................................................

(1)

(e)	The power supply is providing 12 V at 2 A to a circuit connected to its output 			terminals.

(i)	Show that the power dissipated in the MOSFET is approximately 36 W.

................................................................................................................................

................................................................................................................................

(2)

(ii)	To enable the MOSFET to dissipate this power it is bolted to a heat sink.

State three essential features in the design of an efficient heat sink.

................................................................................................................................

................................................................................................................................

................................................................................................................................

(3)

(iii)	The MOSFET is mounted onto a heat sink with a thermal resistance of
			1.1 ºC/W.  If the ambient temperature is 20 ºC, what temperature will the 				MOSFET be at when the power supply is producing 12 V at 2 A?

................................................................................................................................

................................................................................................................................

(2)

(f)	TR1 is included to limit the maximum current that the power supply can provide.

(i)	Explain how this is achieved.

................................................................................................................................

................................................................................................................................

................................................................................................................................

(2)

(ii)	Estimate the maximum output current of the power supply.

................................................................................................................................

................................................................................................................................

................................................................................................................................

(2)

___

(24)

¯¯¯

10).	The diagram below shows the pin connections for a static CMOS RAM IC.

(a)	Calculate how many bytes of data the RAM IC can store.

...............................................................................................................................

...............................................................................................................................

(2)

(b)	By describing the signals that are put onto each of the pins of the IC, explain how a 		byte of data can be written into the IC.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(5)

___

(7)

¯¯¯

11).	A 12 volt lead acid battery is to be kept charged by a solar panel.  The output voltage from 	the solar panel varies from 0V in total darkness to 20V in bright sunlight.  The battery needs 	to be charged at a voltage not exceeding 13.8V.  The circuit diagram is shown below.

(a)	What is the function of diode D1?

...............................................................................................................................

...............................................................................................................................

(2)

(b)	(i)	What is the voltage at point A?

...............................................................................................................................

(1)

(ii)	If the voltage across the battery is 13.8V, what is the voltage at point B?

...............................................................................................................................

(1)

(c)	The op-amp is arranged as a Schmitt trigger.  Briefly state two properties of a 			Schmitt trigger.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(2)

(d)	When the op-amp output is low it has a voltage of zero.  Show that the voltage at 			point E is 6.8V when the voltage across the battery is 13.8V.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(2)

(e)	If the voltage across the battery increases above 13.8V, describe what will happen.

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

...............................................................................................................................

(3)

___

(11)

¯¯¯