Programming and Interfacing revision.

1).	As part of her A level Biology coursework a student is investigating how the temperature of 	a pond varies over a period of a week.  She wants to record the temperatures every hour and 	so designs a computer system to automatically make and record the measurements.  The 	system diagram is shown below.

(a)	The electronic thermometer sensor circuit diagram is shown below.

(i)	When the temperature of the thermistor is 0ºC the voltage at the input of the 			ADC is 0V.  Calculate the resistance of the thermistor?

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	If the voltage at the input of the ADC is 2.5V at 25ºC, calculate the resistance 			of the thermistor at this temperature.

..............................................................................................................................

..............................................................................................................................

(2)

(b)	(i)	State and explain the purpose of the ADC sub-system.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(ii)	Two common ADCs are the flash ADC and the digital ramp ADC.

State one example where the use of each would be appropriate.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	State one major advantage of the flash ADC compared with the digital ramp 			ADC.

..............................................................................................................................

(1)

(iv)	State one major disadvantage of the flash ADC compared with the digital 				ramp ADC.

..............................................................................................................................

(1)

(c)	The student decides to use an 8-bit digital ramp ADC.

(i)	Explain the meaning of the word bit.

..............................................................................................................................

(1)

(ii)	How many different output values are possible from this ADC?

..............................................................................................................................

(1)

(iii)	The output voltage from the thermometer varies from 0V at 0ºC to 2.55V at 			25.5ºC.  What is the minimum change in temperature that can be detected by 			the system?

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(d)	The system diagram for the ADC is shown below.

(i)	In the space below draw the circuit diagram for an 8-bit summing amplifier.

(3)

(ii)	The digital inputs to the summing amplifier are either 0 or 5.0V.  If the output 			voltage from the summing amplifier is –2.5V at a temperature of 25ºC, what 			would be the output voltage at a temperature of 12.8ºC?

..............................................................................................................................

(1)

(iii)	What digital input (in binary) to the summing amplifier is required to give 				this output voltage?

..............................................................................................................................

(1)

(e)	(i)	Draw below the circuit diagram of an analogue inverter, labelling the input, 			the output and suitable component values.

(4)

(ii)	Why is an analogue inverter needed in this application?

..............................................................................................................................

..............................................................................................................................

(1)

(iii)	Explain how the complete ADC functions.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(4)

(iv)	Each input of the input port of the computer has a pull-up resistor and so 				requires inputs to either be 0V or floating.  The output from the op-amp is 				either +12V or -12V.  Explain how the circuit below will interface the output 			of the op-amp to the input port of the computer.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(4)

(f)	During a conversion by the ADC, the computer polls the input port.

(i)	Explain what is meant by the term polled.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	Explain how polling differs from an Interrupt system.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	The ADC is connected to D6 of the input port which is addressed at &H379.

Explain the function of the QBasic statement:-

X% = INP (&H379) AND 64

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(g)	(i)	List the operations that need to be made by the computer for the ADC to 				make a conversion.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(4)

(ii)	Convert this list into a flow diagram.

(4)

(h)	(i)	Explain what is meant by the term subroutine.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	The subroutine to for the ADC to make the conversion is shown below.  By 			each line explain its function.

adcstart:

X% = -1

increase:

X% = X% + 1

IF X% > 255 THEN RETURN

OUT (&H378), X%

Y% = INP (&H379) AND 64

IF Y% = 64 THEN RETURN

GOTO increase

(9)

(j)	The student wants to make temperature measurements every hour for a week.

(i)	Calculate how many hours there are in a week.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	A subroutine called hour transfers control to the subroutine adcstart every 			hour.  What is the QBasic statement for transferring control to the subroutine 			adcstart

..............................................................................................................................

(2)

(k)	The student decides to store the data in an array named DATATEMP.

(i)	Explain what is meant by the term array.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	State, with a reason, the size of this array?

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	What is the QBasic statement for defining this array?

..............................................................................................................................

(2)

(iv)	Where in the QBasic program should this statement appear?

..............................................................................................................................

(1)

(v)	Write a QBasic program to combine together the subroutines mentioned so 			that the system will make measurements every hour for a week and store the 			temperature values in the array datatemp.  The program should ask the user to 			initially enter the start time.  When the measurements have been taken, the 			program should then print to a printer the time and corresponding 					temperature measurements in two columns.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(8)