µ §	Counter, Timer and Logic revision.

1).	A metronome is required that has a speed range from 15 beats per minute up to 240 beats per 	minute, each beat being marked by a short tone pulse of 1000Hz and duration 0.1s.

A proposed system diagram is shown below.

µ §

(a)	(i)	The clock astable is to be made from NAND gates.  Complete the truth table 			for a two input NAND gate, shown below.

|  | Input A | Input B | Output |
| --- | --- | --- | --- |
|  | 0 | 0 |  |
|  | 0 | 1 |  |
|  | 1 | 0 |  |
|  | 1 | 1 |  |

(1)

(ii)	The circuit diagram for a NAND gate astable is shown below.

µ §

By referring to the circuit diagram, explain how a NAND gate astable operates.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(5)

(iii)	Explain why the maximum value for C tends to be 1mF.

..............................................................................................................................

..............................................................................................................................

(2)

(iv)	The slowest speed of the metronome is 15 beats per minute.  What is the 				frequency of the beats?

..............................................................................................................................

(1)

(v)	By referring to the system diagram, state the frequency of the astable needed 			to give this metronome speed.

..............................................................................................................................

(1)

(vi)	If C has a value of 1mF, calculate the combined resistance of the variable 				resistor and the fixed resistor R to give this frequency.

..............................................................................................................................

..............................................................................................................................

(2)

(vii)	The fastest speed of the metronome is 240 beats per minute.  What is the 				frequency of the beats?

..............................................................................................................................

(1)

(viii)	By referring to the system diagram, state the frequency of the astable needed 			to give this metronome speed.

..............................................................................................................................

(1)

(ix)	If C has a value of 1mF, calculate the resistance of the fixed resistor R to give 			this frequency.

..............................................................................................................................

..............................................................................................................................

(2)

(b)	The divide by 4 circuit is made from two D-type flip-flops.  The circuit symbol for a 		D-type flip flop is shown below.

µ §

(i)	Explain the meaning of each of the labelled connections.

S  ..............................................................................................................

..............................................................................................................................

D  ..............................................................................................................

..............................................................................................................................

CK  ...........................................................................................................

..............................................................................................................................

R  ..............................................................................................................

..............................................................................................................................

Q  ..............................................................................................................

..............................................................................................................................

(5)

(ii)	Explain how a D-type flip-flop functions

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	Draw in the space below the circuit diagram for the divide by 4 counter.

(4)

(c)	(i)	The 0.1s monostable is made from NAND gates.  In the space below draw a 			labelled circuit diagram of a NAND gate monostable clearly showing the 				input and the output.

(4)

(ii)	Explain how the NAND gate monostable functions.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(5)

(iii)	If the timing capacitor used in the monostable is 100nF, calculate a suitable 			value for the timing resistor.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(iv)	During the timing period, what is the logic state of the output of the NAND 			gate monostable?

..............................................................................................................................

(1)

(d)	The 1kHz tone is generated by another NAND gate astable.

(i)	In the space below, draw the circuit diagram of a NAND gate astable (without 			looking back to part (a)!).

(3)

(ii)	Calculate suitable component values to give a frequency of 1kHz.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	By considering your answer to part (c)(iv), explain how the circuit diagram 			below will enable 0.1s pulses of 1kHz frequency to be produced.

µ §

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(e)	The output from the circuit above produced a very quiet sound when connected 			directly to a loudspeaker.

(i)	Draw in the space below the circuit diagram of a MOSFET buffer that can be 			used to interface the output of the NOR gate to a loudspeaker.

(3)

(ii)	State three properties of a MOSFET that make it suitable for this application.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

2).	A student wants to construct a circuit that produced pulses of 1 second.  He decides to use 	the mains as a frequency reference and constructs the following circuit to produce pulses of 	approximately 5V amplitude and frequency 50Hz from a 12V alternating supply.

µ §

(a)	(i)	Explain why the output consists of pulses at a frequency of 50Hz.

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	What is the period of the pulses?

..............................................................................................................................

(1)

(iii)	Explain why the pulses have an amplitude of approximately 5V.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(b)	The system diagram below shows how the 50Hz pulses are converted into 1 second 		pulses.

µ §

(i)	Explain what is meant by the term mark to space ratio.

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	The diagram below shows two rising edge triggered D-type flip-flops.  				Complete the circuit diagram below to show how they can be connected to 			form a divide by four up-counter.  Label the input and the output.

µ §

(5)

(iv)	Explain how this circuit functions.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(v)	Complete the timing diagram below for the Q output of A and the 
			Q output of B.

µ §

(c)	The diagram below shows three rising edge triggered D-type flip-flops.  				(i)	Using any additional components that you require, complete the circuit 				diagram to show how they can be connected to form a divide by five 
			up-counter.  Label the input and the output.

µ §

(5)

(ii)	Explain how this circuit functions.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(iii)	Complete the timing diagram below for the Q outputs of A, B and C.

µ §

(4)

3).	As part of a game for a fair, a display board is required that has 4 lights flashing in sequence, 	in a square, at the edge of the board.  A student is asked to construct this lights effect and 	decides to use a four bit shift register.

(a)	Explain what is a shift register.

..............................................................................................................................

..............................................................................................................................

(1)

(b)	The shift register is to be constructed from rising edge triggered D-type flip-flops.

Explain the meaning of the following terms:

(i)	rising edge triggered,

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	D-type flip-flop.

..............................................................................................................................

..............................................................................................................................

(1)

(c)	Complete the circuit diagram below to show how the four D-type flip-flops are 			connected to form a suitable shift register.  Label the clock input and the serial data 		input.

µ §

(5)

(d)	A 12V 24W lamp is to be connected to each Q output via a buffer circuit.

(i)	Explain why the buffer circuit is needed.

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	A transistor or a MOSFET could be used as the buffer.  State three reasons 			why a MOSFET is more suited to this application.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(iii)	Draw below the circuit diagram of a MOSFET buffer for this application.

(3)

(d)	Only one lamp is to be lit at any time and so the student sets up a truth table for the 		serial data input as shown below.

| Data input (D) | QA | QB | QC | QD |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |

(i)	State the Boolean expression for D.

..............................................................................................................................

(1)

(ii)	Draw below a circuit diagram to implement this expression, labelling the 				inputs and output.

(3)