Course Questions

These questions have been written to accompany the Foundation and Further Electronics Booklets written by Ian Kemp for the AQA AS Electronics Syllabus.

Each section is referenced to the relevant pages in the booklets.

These questions are not meant to be past paper practice for exam revision, they are designed to take the learner through the basics section by section.

J. Brock

Contents

Page		Title

3. System Synthesis

4. Logic Systems (Foundation)

5. Basic Practical Principles

6. Making Any Gate with NANDs

8. Three input and Combinations of Gates

9. Simple Boolean Expressions

10. Deriving Boolean Expressions from a circuit

11. Karnaugh Maps

13. Going from Truth Table to Karnaugh Map

14. Further Boolean Expressions

16. Further Karnaugh Maps

17. Monostables

18. Astables

19. 555 Timer Circuits

20. Bistables

22. Data Latches

23. D Type Counters

25. Zener Diodes

26. Diodes

27. Resistance

28. More Resistance

29. Voltage, Current and Resistance

31. Potential Divider Equation and Sensors

32. Voltage, Current and Resistance (again)

34. Voltage, Current and Power

35. Perfect Op Amps and Comparators

39. Other Op Amp Circuits

41. Summing Amplifiers

43. Capacitors, Music and Filters

45. The Four Active Filter Circuits

50. FETs and Push Pull Amplifiers

51. Heatsinks and Thermal Resistance

System Synthesis

Pages 7, 8 and 9 Foundation Electronics, Kemp

- Identify the input sensor, the process, the output device and the feedback loop for the systems below.

- Temperature control inside the Blackbird reconnaissance aircraft which can get as high as 600 C on the outside but must stay below 25C inside.

- Water level control in an industrial boiler.

- Submarine auto depth control

- Using the basic ideas of any control system i.e. input, desired value, process, output and feedback draw the block diagrams for the following systems and describe how each one works.

- Aircraft automatic pilot altitude control

- Cruise control in a car

# Logic Systems

Pages 10 to 13 Foundation Electronics, Kemp

- What logic functions are necessary for the following systems? In each case define the logic levels and say how the system works using a truth table in the explanation.

- A laser safety systems that turns off a dangerous rotating machine if the beam is cut.

- An alarm that sounds if any one of four doors is opened.

- Shop security system that has a wire through the handle of each portable stereo so that if it is taken from the shelf the wire breaks and the alarm sounds.

- Name the gate and draw the symbol for the truth tables below.

a

A   B     F

0    0     0

0    1     0

1    0     0

1    1     1

b

A   B     F

0    0     0

0    1     1

1    0     1

1    1     1

c

A   B     F

0    0     0

0    1     1

1    0     1

1    1     0

d

A   B     F

0    0     1

0    1     1

1    0     1

1    1     0

e

A   B     F

0    0     1

0    1     1

1    0     1

1    1     0

- In digital systems we deal with 1s and 0s. With a switch on may be 1 and off may be 0.

- With 2 switches how many different possibilities are there?

- With 3 switches how many different possibilities are there?

- Draw a table with columns ‘Number of Switches’ and ‘Possibilities’ upto 4 switches. Can you see a relationship between the number of switches and the number of possibilities?

- How do you from the symbol that the gate name usually begins with an N?

- What is the simplest gate?

- What is its truth table?

Basic Practical Principles

No specific pages of Kemp.

- On most 4000 series quad gate ICs which pins are connected to the +V and 0V?

- What range of supply voltages is possible with the 4000 series?

- What is the disadvantage of the 4000 series?

- What 4 things are wrong with the circuit below?

- Redraw the circuit with all the corrections made and explain why you have made them.

- When is a logic one input to the gate?

- When is a logic 0 input to the gate?

- What is the logic level of an input that’s not connected?

Making any gate with NAND gates

Page 13 Foundation Electronics, Kemp

- Look at the truth table below for a NAND gate. If we connect the 2 inputs together then they must always have the same logic level as each other.

A   B     F

0    0     1

0    1     1

1    0     1

1    1     0

- What 2 input conditions can you now cross out from the table?

- Look at the table again with these crossed out. What has the gate become effectively?

- (a)      Draw the truth table for a NAND gate.

(b)	     Above the output column draw a small symbol for a NAND gate.

- Now add just one more column to the right of the table showing the output for an AND gate.

- Draw a small symbol for an AND gate above this column.

- What does adding the nose to the symbol do to the output?

- Bearing in mind question 1 can you make an AND gate using 2 NANDs?

- Look at the circuit below.

(a) By labelling the outputs from the three NAND gates P, Q and R complete the truth table below.

A    B        P    Q     R

0     0

- 1

- 0

1     1

(b)      Cross through the columns P and Q. What simple gate do these   three make?

- How could you make a NOR gate with NANDs?

- Why bother?

Three input and combinations of gates

Pages 13and 14 Foundation Electronics, Kemp

20.  Draw the truth tables for three input AND, NAND, OR, NOR, XOR and XNOR.

21. Copy and complete the truth table for each of the circuits below having a column for each gate output i.e. A, B, C D, E and F.

| A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  |  |  |
| 0 | 0 | 1 |  |  |  |
| 0 | 1 | 0 |  |  |  |
| 0 | 1 | 1 |  |  |  |
| 1 | 0 | 0 |  |  |  |
| 1 | 0 | 1 |  |  |  |
| 1 | 1 | 0 |  |  |  |
| 1 | 1 | 1 |  |  |  |

(a)

(b)

(c)

Simple Boolean Expressions

Pages 14 to 17 Foundation Electronics, Kemp

22. Draw the gates for the following boolean expressions

_

(a) A

(b) A.B

(c) A+B

___

(d) A.B

___

(e) A+B

23. The following boolean expressions cannot be made from a single gate. Design a circuit for each expression below :

(a) with any gates that you choose and

(b) from NAND gates only.

_ _

(i) A.B

_ _

(ii) A+B

_

(iii) A.B

_

(iv) A.B

_

(v) A+B

_

(vi) A+B

You should have drawn 12 circuits.

Deriving Boolean Expressions from a Circuit

Pages 7 to 9 Further Electronics, Kemp

The easiest way to do this is to construct a truth table with a column for each gate output then identify the expression from the final output column.

To do this

(i) highlight the rows that have a 1 output

(ii) If an input is a 0 for this row then write it down as a barred input,

(iii) If an input is a 1 for this row then write it down as normal

(iv) In between each input put a . i.e. the AND function

(iv) finally combine the different row expressions with + as the OR function.

For example consider the section of truth table below:

| A | B | C | D |  | F | Expression |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |  | 1 | _ _ _ A.B.C.D |
| 0 | 1 | 1 | 1 |  | 1 | _ A.B.C.D |
| 0 | 1 | 1 | 0 |  | 0 | No expression |

The function then becomes:

_ _   _     _

F = A.B.C.D + A.B.C.D

24. Derive the boolean expressions for the circuits below:

(a)						(b)

(c)						(d)

# Karnaugh Maps

Pages 10 to 16 Further Electronics, Kemp

When using Karnaugh maps you need to follow some simple guidelines.

1. Notice the different way of laying out the numbers (Gray Code)

2. Loop (or in this case shade as looping is difficult on the computer) groups of 1s in 2s, 4s and 8s. No diagonals.

3. An inputs value it shown in the bold squares at the edges of the table.

4. If an input changes its value within a loop it goes out of the expression.

5. If an input stays as a 1 it remains in the expression as non barred.

6. If an input stays as a 0 it remains in the expression as barred.

7. Karnaugh maps loop round on themselves so a 1 in each corner can become one group of 4.

Example:

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 0 | 0 | 1 |
| 01 | 0 | 0 | 0 | 1 |
| 11 | 0 | 0 | 0 | 1 |
| 10 | 0 | 0 | 0 | 1 |

The lighter shaded group of 4 1s can be simplified.

1. B remains in the expression as B because it remains at 1 in all squares within the shaded group.

_

2. A remains in the expression as A  because it remains at 0 in all squares within the shaded group.

3.   C goes out as it changes value as you go down the shaded group.

4. D goes out for the same reason.

_

So the lighter shaded group becomes A.B

The darker shaded pair can also be simplified as the maps loops round.

We need to look at the first and last columns.

_

1. Here A remains at 0 so stays in as A.

2. B goes out as it is 0 in the first column but 1 in the last.

_ _

3. C and D both stay in as C.D as this shaded area is only in the first row and both C and D stay as a 0.

_  _ _

So this group becomes A.C.D		      _      _ _ _

Overall then this function becomes:     F = A.B + A.C.D

25. Try simplifying the following Karnaugh maps.

(a)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 0 | 0 | 1 |
| 01 | 0 | 0 | 0 | 0 |
| 11 | 0 | 0 | 0 | 0 |
| 10 | 1 | 0 | 0 | 1 |

(b)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 1 | 0 | 0 |
| 01 | 0 | 0 | 1 | 1 |
| 11 | 0 | 0 | 0 | 0 |
| 10 | 1 | 1 | 0 | 0 |

(c)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 1 | 1 | 0 |
| 01 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 0 | 0 |
| 10 | 0 | 1 | 1 | 0 |

(d)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 0 | 0 | 1 |
| 01 | 1 | 0 | 0 | 1 |
| 11 | 1 | 0 | 0 | 1 |
| 10 | 1 | 0 | 0 | 1 |

(e)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 1 | 0 |
| 01 | 1 | 1 | 1 | 1 |
| 11 | 0 | 0 | 1 | 0 |
| 10 | 0 | 0 | 1 | 0 |

(f)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 1 |
| 01 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 0 | 0 |
| 10 | 0 | 0 | 0 | 1 |

(g)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 1 | 0 | 0 |
| 01 | 1 | 1 | 0 | 0 |
| 11 | 0 | 0 | 0 | 0 |
| 10 | 0 | 0 | 0 | 0 |

(h)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 1 | 1 |
| 01 | 0 | 0 | 1 | 1 |
| 11 | 0 | 0 | 1 | 1 |
| 10 | 0 | 0 | 1 | 1 |

(i)

| BA DC | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 1 | 1 | 1 |
| 01 | 0 | 0 | 0 | 0 |
| 11 | 0 | 0 | 0 | 0 |
| 10 | 1 | 1 | 1 | 1 |

Going from Truth Table to Karnaugh Map

Pages 10,11 and 12 Further Electronics, Kemp

26. Convert the following truth tables to Karnaugh Maps and simplify the expression. Be careful, the Karnaugh Maps follow a gray code so you’ll need to think what the input values are for each square.

(a)

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

(b)

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

(c)

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

(d)

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 |

(e)

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 |

(f)

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

(g)

| A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

(h)

| A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

(i)

| A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Further Boolean Expressions

Pages 10 to 17 Foundation Electronics, Kemp: Pages 7 to 16 Further Electronics, Kemp

Below are some logic truth tables. For each one:

- Write out the full Boolean expression,

- Minimise the Boolean expression,

- Draw out a logic circuit that could fulfil this function using different gates,

- Draw out the logic circuit that could fulfil this function using only NAND gates.

27.

| A | B | F |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

28.

| A | B | C | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

29.

| # A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Further Karnaugh Maps

Pages 10 to 17 Foundation Electronics, Kemp

Pages 7 to 16 Further Electronics, Kemp

For each of the specifications below draw out the Karnaugh map and produce a circuit using NAND gates only to satisfy requirements.

- The shuttle has a voting system whereby 3 computers vote on what to do. The final decision always goes with the majority so that if one computer goes down the other 2 function normally. So if 2 or more computers vote 1 then the output will be a 1. If 2 or more vote 0 the output will be 0.

- A system is required for a fighter jet that will only eject the pilot if the eject button is pressed and the cockpit glass has gone. In addition there is an altitude detector which gives a ‘0’ if the aircraft is above 500ft and a ‘1’ if it’s below 500ft. The eject system needs at least 500ft for the pilot to eject safely.

- In a bank hierarchy it is only possible for junior members of the bank to open the safe if given permission by the manager above him. This also applies to the manager above his manager. A ‘1’ opens the safe and a person attempting to open the safe inputs a ‘1’. The manager giving permission for an underling to open the safe also inputs a ‘1’. Design the system for 2 managers and one poor underling.

Monostables

Pages 51 to 53 Foundation Electronics, Kemp

Page 21 Further Electronics, Kemp

This is a simple capacitor circuit.

- Draw the voltage time graphs for the voltage across the resistor and the voltage across the capacitor starting from the time that the switch has just been moved to its ‘UP’ position as shown.

- What determines how quickly the capacitor charges?

- What will happen when the switch returns to its ‘DOWN’ position?

- Draw the voltage time graphs for the voltages across the resistor and the capacitor starting from the time when the switch has just changed.

- Try building this circuit on crocodile clips and select the oscilloscope to see if your answers are right.

This is a Monostable Circuit built from NANDs

- Write down the truth table for a NAND gate.

- Using this and graphs (similar to the ones you have drawn above) describe the operation of the above monostable.

- What is the time of the pulse produced?

- The LED is on all the time unless the monostable is triggered. Once triggered the LED goes off for a time. Design a monostable that turns the LED on for a time.

Astables

Page 22 Further Electronics, Ian Kemp.

Astables can also be made from Logic gates. Below is the simplest logic Astable. Careful though Crocodile Clips doesn’t like it, but it does work in real life. If you get the chance in the exam use this one but make sure you make the point that it needs a Schmitt logic gate to work.

- Draw the truth table for an invertor.

- By describing the logic levels at various places and considering how a capacitor charges and discharges explain how the circuit works.

- Design the same circuit but this time only use:

- a NAND gate

- a NOR gate

- Why might the circuit stop functioning if the resistor is made too small?

- Below is a more likely Astable that you’d face in the exam. It’s more complicated than the one above. Listen to your teacher’s explanation of it and then using truth tables and graphs describe how it works.

- There’s an unmarked input to the right of the circuit what does do?

- Explain how it does it.

- If the formula for this circuit is:

F = 1/2RC

What is the frequency of the output pulses?

555 Timer Circuits

Page 54 Foundation Electronics, Kemp

- (a) Calculate the time period for the single pulse produced from the circuit below.

(b) What is the purpose of the 10K resistor?

(c) What happens if you press the trigger button and hold it down?

- Calculate :

  - the high time;

  - the low time

  - and the frequency of pulses from the output of the astable circuit below.

- Design an astable circuit with produces a pulse every 20ms.

- Design a monstable circuit that produces a pulse that can be varied from 1ms to 2 ms.

- Using the 2 circuits above design a combined circuit that produces a pulse every 20ms that can be varied between 1ms and 2ms.

Bistables

Page 17 Further Electronics, Kemp

Below is a simple logic bistable

Circuit 1

- Draw the truth table out for a NAND gate.

- By stating the logic levels at various places explain how the bistable (or latch) in Circuit 1 works.

- Circuit 2,  below, is the same circuit as Circuit 1 above. It is sometimes drawn this way in exams. Don’t be phased the same explanation applies.

Circuit 2

- Circuit 3, below, is another latch. Draw the truth table out for a NOR gate and explain the circuits operation in the same way.

These types of latch are very basic and it’s difficult to synchronise several at a time. For that type of application you need a D type latch (or flip flop).

Data Latches

Pages 17 to 20 Further Electronics, Kemp

Dtype latches can also be used as the fundamental data latch used in the output from a microprocessor system.

Data Latch

Data appears at these outputs when the clock input goes high. The data is latched when the clock input returns low.

The micro- processor would apply data to these inputs.

Clock input

Shift Register (Serial to Parallel)

Q0	       Q1	      Q2	   Q3

Clock I/P

Data I/P

Data from the serial data input is clocked into the register on successive clock pulses. This then becomes available on Q0 to Q3 as parallel information.

- Draw the timing diagram (similar to that on page 19 of Further Electronics) for the 4 bit piece of data 1011 as it’s put into the register.

Shift Register (Parallel to Serial)

- Where might this circuit be useful?

- Explain how it works.

D type Counters

Pages 24 to 31 Further Electronics, Kemp

# Drawing and identifying counters

- The Q output always feeds back to the D input.

- The Q output is always displayed i.e. it’s connected to a lamp.

- If the Q output is propagated then the counter is DOWN counter (change +ve edge).

- If the Q output is propagated then the counter is an UP counter (change –ve edge).

- If you want a ‘divide by’ counter then join the outputs that correspond to a 1 for that number to the input of an AND gate and feed the output of the AND gate back to all Resets of the D types.

They all have this part in common.

1. This is a DOWN counter, draw its timing diagram.

2. This is an UP counter, draw its timing diagram

3. This is a divide by 5 UP counter, draw its timing diagram

4. Draw a divide by 9 DOWN Counter

Reset O/P

5. Draw a divide by 10 UP Counter

# Zener Diodes

Pages 29 to 30 Foundation Electronics, Kemp

67. Draw the characteristic V/I graph for a zener diode.

68. A 600mW 8.9V zener diode is used in the circuit below to give a steady supply to a hifi system from a 12V car battery.

(a) Calculate the maximum current that can flow through the zener diode.

(b) The zener requires 3mA to flow through it to maintain its voltage. What is the maximum current that can be used by the load?

(c) What is the value of the voltage across R?

(d) Calculate the minimum value of R.

(e) Calculate the required power rating of the resistor

(f) Why might a small resistor be placed across the zener diode?

69. A 800mW 5.1V zener diode is used in the circuit below to give a steady supply to an alarm system from a 12V car battery.

(a) Calculate the maximum current that can flow through the zener diode.

(b) The zener requires 5mA to flow through it to maintain its voltage. What is the maximum current that can be used by the load?

(c) What is the value of the voltage across R?

(d) Calculate the minimum value of R.

(e) Calculate the required power rating of the resistor

Diodes

Page 26, Foundation Electronics, Kemp

- A red LED has a forward bias voltage of 1.8V and is to be powered from a 9V supply.

  - The LED does not light. Why?

  - What is the voltage drop across the resistor?

  - If the maximum current through the LED is 10mA what value of resistor is required?

- Draw the V/I characteristic for a diode and explain what it means.

- In the circuit below the transistor driving the motor keeps blowing up.

  - By discussing the nature of switching electromagnetic devices on and off explain what is going on.

  - Explain how a diode can be used to prevent it happening again.

Resistance

Pages 20-22 Foundation Electronics, Kemp

73. Calculate the resistance in the following circuits.

(a)					(b)				(c)

74. Calculate the resistance in the following parallel circuits

(a)				      (b)				 (c)

(d)				    (e)					 (f)

(g) What do you notice about the total resistance as more resistors are added in parts (d) to (f)?

More Resistance Questions

75. Calculate the resistance in the following circuits.

(a)					(b)				(c)

76.  Calculate the resistance in the following parallel circuits.

(a)				      (b)				 (c)

Voltage, Current and Resistance

Pages 18 –23 Foundation Electronics, Kemp

Voltage Rule 1: Voltages across components in a purely series circuit add up to the battery or supply voltage.

Voltage Rule 2: Voltage across each loop in a purely parallel circuit is the same as the battery or supply voltage.

Voltage Rule 3: If there’s a combination of parallel and series circuits it’s sometimes helpful to calculate the parallel combinations first then treat them as single resistances.

77.     (a)What is the voltage across each resistor in the circuit below?

(b) What is the total resistance of the circuit?

(c) What is the total current drawn from the battery?

78. In the circuit below the current through the 10K resistor is 2mA

(a) What is the voltage across the 10K resistor?

(b)What is the voltage across the 15K resistor?

(c) What is the battery voltage?

(d) What is the current through the 16K resistor?

(e) What is the total resistance of the circuit?

79.  In the circuit below

(a) Calculate the total current drawn from the battery.

(b) Calculate the current flowing in each resistor.

80. Redraw the circuit below showing:

(a) where you would connect voltmeters to find the voltage across each resistor.

(b) where you would connect ammeters to find the current through each resistor.

(c) Calculate the resistance of the parallel combination of the 8K and 15K.

(d) Calculate the resistance of the parallel combination of the 16K and 3.3K.

(e) Find the total resistance of the circuit.

(f) Calculate the total current in the circuit.

(g) Calculate the voltage across each resistor.

Potential Divider Equation and Sensors

Pages 31 – 35 Foundation Electronics, Kemp

81.  For each of the circuits below use the potential divider equation to calculate the voltage across each resistor.

(i)					(ii)				(iii)

(iv)

Remember give a resistive input transducer what it wants and its resistance goes down.

82. In the temperature sensing circuit below calculate the voltage across the thermistor at 0*C and 60*C.

Resistance/ohms

105

104

103

102

10

0    20   40    60   80   100   Temp/ *C

83. Using the same thermistor characteristic curve as in question 2 calculate the voltage across the resistor when it is 20*C and 40*C for the circuit below.

84. In the light sensing circuit below calculate the voltage across the LDR at 1 lux and  100 (102) lux.

Resistance/ ohms

106

105

104

103

102

0.1   1    10   102  103    104

85. Using the same characteristic LDR in question 4 calculate the voltage across the resistor in the circuit below when the light is 0.1 lux and 10,000 (104) lux.

86. Design a logic system that will:

(a) turn the lights on when it gets too cold or too dark.

(b) sound an alarm if it’s light or hot.

(c) Unlock a door only if it’s light and hot or if a button is pressed.

# Voltage, Current and Resistance (again)

Page 20 Foundation Electronics, Kemp

## V=IR

87. Find the voltage across and current through each component in the circuits below.

(a)					(b)				   (c)

(d)				   (e)					(f)

# Voltage, Current and Power

Page 20, Foundation Electronics, Kemp

88. Calculate the voltage, current and power across each component in the circuit below.

(a)	          			      (b)				     (c)

(d)				   (e)					 (f)

Perfect Op Amps and Comparators

## Page 43, Foundation Electronics, Kemp

- An ideal op amp has 5 main characteristics.

  - List each characteristic.

  - For each characteristic explain what it means.

  - For each characteristic explain what a true value is in a real op amp.

- The gain of an op amp drops off as frequency increases. In fact gain x frequency is a constant. If the gain of an op amp is 1,000,000 at a frequency of 10Hz what is the gain at:

  - 100Hz

  - 1000Hz

  - 10kHz

  - 100Khz?

- Describe the action of a comparator in terms of V into inverting input (V2), V into the non-inverting input (V1) and Vout.

- Consider the circuit below.

  - Using the potential divider equation calculate the voltage at the non-inverting input.

  - Using the potential divider equation calculate the voltage at the inverting input.

  - Will the LED be on or off?

  - The resistance of the LDR changes with light conditions. At what resistance of the LDR will the output change?

  - Is this a too dark or a too light warning lamp?

- Consider the circuit below:

  - Calculate the voltage at the inverting input.

  - Calculate the voltage at the non-inverting input.

  - Is the LED on at the moment?

  - What happens to the thermistors resistance as the temp rises?

  - At what value of resistance of thermistor will the output change?

  - Is this a high temp warning light or a low temp warning light?

- The circuit below is designed to operate a 500W lamp in a car park when it gets too dark.

  - Calculate the voltage at the inverting input.

  - Calculate the voltage at the non-inverting input.

  - Is the output from the comparator high or low?

  - Why is the relay necessary?

  - Why is there a diode across the relay?

- In this example we are working on a dual rail power supply so you have to pay attention to the minus volts.

  - What is the total voltage drop across the 22K and 47K resistors?

  - Calculate the voltage across the 47K resistor. Note this is the voltage above –9V.

  - Calculate the voltage at the inverting input above 0V.

  - Calculate the voltage drop across the LDR.

  - Calculate the voltage of the non-inverting input above 0V.

  - Is the lamp on or off?

- Consider the circuit below:

  - Calculate the voltages at the inverting inputs of the bottom, middle and top op amps.

  - At what input voltages will the Green, Yellow and Red LEDs come on?

  - Modify the circuit to become a temp sensor that displays cold medium and hot.

  - Explain how your new circuit works.

Other Op Amp Circuits

Page 34, Further Electronics, Kemp

- What is the gain of the following circuits?

(a)						(b)

(c)							 (d)

- What is the gain of the circuits below?

(a)							(b)

- (a) What is the formula for the gain of a non-inverting amplifier?

(b) What is the value of the feedback resistor here?

(c ) What is the value of Rin here?

  - What is the gain of this circuit?

  - What is the point of this circuit?

Summing Anplifiers

Page 39 –40, Further Electronics, Kemp

100. (a) What is the formula that relates Current (I) to Voltage (V) and resistance (R)?

(b) What is the input resistance of a perfect op amp?

(c) In the circuit below how much of the currents that enter through R1, R2 and R3 flow into the inverting input?

(d) Where do these currents go?

- Write down all these currents in the format in part (a) i.e. in terms of V and R.

- Rearrange these currents to give Vout as the subject of the equation.

- Why is this circuit called a summing amplifier?

- (a) In the circuit below if V1 = 2V, V2 = 0.5V and V3 = 0.85V what is the value of Vout?

(b) Which voltage makes the biggest contribution to the output?

- Using the circuit in question calculate the missing values in the table below. Assume supply voltage is +-15V.

| R1 | R2 | R3 | Rf | V1 (V) | V2 (V) | V3 (V) | Vout (V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10K | 10K | 10K | 10K | 1 | 2 | 3 |  |
| 12K | 36K | 22K | 10K | 1.5 | 2 | 3.3 |  |
| 100K | 120K | 180K | 220K | 0.2 | 0.5 | 0.6 |  |
| 1K | 2K | 3K | 10K | 2.4 | 7.7 | 8.9 |  |
| 22K | 82K | 10K | 10K | 1.2 | 3.6 | 0.7 |  |

Capacitors, Music and Filters

Page 41, Further Electronics, Kemp

Capacitors have a resistance that changes or reacts to the frequency of a signal, for instance music. So instead of calling it resistance it’s called capacitive reactance, Xc, measured in ohms.

If you connect a capacitor to a d.c. voltage current drops to zero after a while, i.e. the capacitors reactance is very high to d.c.  i.e. to 0Hz. The opposite happens at high frequency i.e. the capacitors reactance to high frequency is very low.

So high frequency means low reactance and low frequency means high reactance. It’s like opposites, one goes up the other goes down.

In fact:

Xc = 1/2fC

- Calculate the capacitive reactance and fill in the table below:

| Capacitance uF | Frequency Hz | Reactance  |
| --- | --- | --- |
| 100 | 1000 |  |
| 100 | 10,000 |  |
| 2200 | 100,000 |  |
| 0.22 | 2000 |  |
| 0.22 | 200 |  |

- (a) Draw the circuit for a passive low pass filter with a 100uF capacitor and a 10K resistor.

(b) Calculate the cut off frequency for this filter.

(c) What is the value of gain (Vout/ Vin) at this frequency?

(d) Draw the frequency response of this filter.

- (a) Draw the circuit for a passive high pass filter with a 220nF capacitor and a 1K resistor.

(b) Calculate the cut off frequency for this filter.

(c) What would you hear if you played music through this filter?

(d) Draw the frequency response of this filter.

- (a) What is the formula for the gain of an inverting amp?

(b) What is the reactance of a capacitor at Low frequency?

(c) What would be the gain of an inverting amplifier that had a capacitor as the feedback resistance (Rf) at low frequency?

(d) What would the gain of this amplifier at high frequency?

(e) Explain your answer to (d).

(f) If you played music through this amplifier what would it sound like?

- (a) What is the reactance of a capacitor at high frequency?

(b) What would be the gain of an amplifier that had a capacitor as an input resistance (Rin) at high frequency?

(c) What would be the gain of this amplifier at low frequency?

- Explain your answer to (c).

- If you played music through this amplifier what would it sound like?

- An amplifier with a capacitor in the feedback loop to replace Rf has a problem if a d.c. voltage is applied at its input, even if that voltage is very small.

  - What is the reactance of a capacitor at d.c (i.e. 0Hz)?

  - Explain what the problem is using the gain formula for an inverting amp in your answer.

  - This problem can be overcome by placing a resistor in parallel with the feedback capacitor. Why does this help?

- An amplifier with a capacitor in the place of the input resistance (Rin) has a problem if a very high frequency signal is put into the amp.

  - What is the reactance of a capacitor at high frequency?

  - Explain what the problem is using the gain formula for an inverting amp in your answer.

  - This problem can be overcome by placing a resistor in series with the input capacitor. Explain how this helps.

### The Four Active Filter Circuits

Page 45, Further Electronics, Kemp

These are based on the inverting amplifier which as you know has a gain controlled by the resistances around the op amp.

Gain = -Rf/Rin

Now what would happen if one of those resistances was a capacitive reactance, i.e. a resistance that changes with frequency?

The gain of the amplifier would different at different frequencies.

The cut off or break frequency of a filter occurs when the reactance of the capacitor equals the resistance of the resistor associated with it.

fo = 1/2RC

Another useful thing to remember here is that the resistance of 2 resistors in parallel is always less than the smallest resistor.

Look at the treble cut filter below.

  - What will the reactance of the capacitance at low frequency?

  - If Xc is much more than 1M what is the effective resistance of the parallel combination?

  - What is the gain of this amplifier at low frequency?

  - What is the reactance of the capacitor at high frequency?

  - What will happen to the gain of this amplifier as frequency increases?

  - Calculate the cut off frequency of this filter.

  - Draw the frequency response graph for this filter. Try to include as much numerical information as you can.

- Consider the treble boost filter circuit below.

The input resistance in this amplifier is a parallel combination of the capacitor and 56K resistor. In a parallel combination the total resistance is always less than the smaller one.

  - Calculate the cut off frequency of this filter.

  - What will the reactance of the capacitor be at low frequencies?

  - What is the approximate resistance of the input parallel combination at low frequencies?

  - Calculate the gain of the filter at low frequencies.

  - What will the reactance of the capacitor be at high frequencies?

  - What will happen to the value of the input parallel combination as frequency increases?

  - Explain what happens to the gain of this amplifier as frequency increases.

  - Draw the frequency response graph for this filter.

  - What will be the gain of this amplifier for very high frequency noise?

  - Why is another resistor often added in series with the input combination?

Consider the bass cut filter circuit below.

The input resistance in this amplifier is a series combination of the capacitor and 100K resistor. In a series combination the total resistance is the sum of the resistors.

  - Calculate the cut off frequency of this filter.

  - What will the reactance of the capacitor be at high frequencies?

  - What is the approximate resistance of the input series combination at high frequencies?

  - Calculate the gain of the filter at high frequencies.

  - What happens to the reactance of the capacitor as frequency decreases?

  - What will happen to the value of the input series combination as frequency decreases?

  - Explain what happens to the gain of this amplifier as frequency decreases.

  - Draw the frequency response graph for this filter with as much numerical information as possible.

In the Bass Boost filter circuit below the feedback  resistance has been replaced by a series combination of a capacitor and a 10K resistor.

In a series combination the total resistance is the sum of the resistances.

  - Calculate the cut off frequency of this filter.

  - What will the reactance of the capacitor be at high frequencies?

  - What is the approximate resistance of the feedback series combination at high frequencies?

  - Calculate the gain of the filter at high frequencies.

  - What happens to the reactance of the capacitor as frequency decreases?

  - What will happen to the value of the input series combination as frequency decreases?

  - Explain what happens to the gain of this amplifier as frequency decreases.

  - Draw the frequency response graph for this filter with as much numerical information as possible.

  - What would be the gain of this filter at dc?

  - A 10M resistor been added to the circuit as below. What does this do to the gain of the circuit at dc?

In the circuit below the two ideas are combined together so that you have both a bass boost filter and a treble boost filter. This is a band pass filter.

  - Explain why the 10nF capacitor and the 100K resistor form a  treble boost filter.

  - Calculate the cut off frequency for the treble boost filter.

  - The 1nF capacitor and the 100K parallel resistor form a bass boost filter. Calculate its cut off frequency.

  - Draw a frequency response curve (Gain against frequency) for this filter showing the cut off frequencies.

FETs and Push Pull Amplifiers

Foundation Electronics, Kemp page 38

Further Electronics, Kemp page 47

List and explain  three advantages of a MOSFET over a junction transistor.

Unlike a junction transistor the MOSFET requires a resistor connected from its gate to 0V.

  - What is the approximate value for this resistor normally?

  - What does this resistor do?

Draw the Vout/Vin characteristic for a MOSFET.

Design a circuit that drives a fan motor when it gets too hot. The circuit should include a MOSFET.

(a) Draw the circuit for a simple push pull amplifier using MOSFETs.

- With reference to the Vout/ Vin  characteristic for a MOSFET describe crossover distortion and what causes it.

- What must be added to the amplifier to reduce this distortion?

- To improve the power amplifier further an op amp is added to the circuit with the output fed back from the MOSFET outputs. Why is this an improvement?

Heatsinks and Thermal Resistance

Further Electronics, Kemp Page 52

Power amplifiers get hot but their maximum operating temperature should not exceed 1000C and unless otherwise stated the normal air temperature is assumed to be as hot as it gets approx 350C. Their temperature increase per watt of power dissipated is called their thermal resistance.

- Describe four ways in which a semiconductor can be kept cool.

- If the maximum current passing through a MOSFET is 10A when there is a drain to source voltage of 6V:

  - Calculate the power dissipated in the MOSFET

  - Calculate the maximum allowable temperature rise.

  - Calculate the required thermal resistance of the heat sink.

- Describe thermal runaway in junction transistors and explain why it is not a problem in MOSFETs